Abstract

There are many interesting aspects of mobile robotics: inputs, outputs and control. This project investigates all these areas to some extent for current research and possibilities, with the aim of developing a low cost, easy to build and yet seemingly intelligent robot.

The platform chosen to build the mobile robot is Lego, which makes construction very easy. The control mechanism chosen for the project is Brooks’ subsumption architecture, a layered reactive control architecture that has had significant success in many robotics applications.

The first three layers of a subsumption architecture are implemented using LeJOS, a Java virtual machine which runs on the Lego RCX. These layers progressively allow the robot to avoid objects, wander randomly and then hunt down a tennis ball. A ball sensor is constructed from an array of range finders and some comparison logic circuitry to reason about the location of the ball.

The project achieves limited success. When placed in an arena with a ball, the robot successfully locates the ball, albeit after some time. If the robot is presented with a moving ball it is too slow to track it, either in turning or following, and hence fails to catch it.

Table of Contents

Introduction	6

Literature Review	7

2.1	Input Sensors	7

2.1.1	Range finders	8

2.1.2	Cameras	9

2.2	Robot Movement	10

2.2.1	Legs	10

2.2.2	Wheels	10

2.2.3	Tracks	10

2.3	Robot Construction	11

2.4	Lego	11

2.4.1	Lego Mindstorms	11

2.4.2	Alternative RCX Firmware	12

2.4.3	Lego Sensors	13

2.4.4	Multiplexing	14

2.4.5	Interfacing with Range Finders	17

2.5	Architectures for Intelligence	17

2.5.1	Logic Based Agents	18

2.5.2	Belief-Desire-Intent Agents	19

2.5.3	Reactive Agents	19

2.5.4	Subsumption Architecture	21

2.6	Existing Projects and Work	23

2.6.1	Subsumption Architecture in LeJOS	23

2.6.2	Tennis Ball Catcher	23

2.6.3	Brooks’ Robot	24

2.6.4	Map Building with a Lego Robot	25

2.7	Problem Definition	25

Design 	26

3.1	Design Constraints and Limitations	26

3.2	Lego Robot Platform	26

3.3	RCX Software Design	29

3.4	Subsumption Architecture	30

3.5	Ball Sensor Design	32

3.6	Summary	33

Implementation	34

4.1	The Ball Sensor	34

4.1.1	Comparators	34

4.1.2	The Logic Stage	35

4.1.3	Buffering	35

4.1.4	Testing	36

4.2	The Robot	37

4.3	Software Implementation	38

4.3.1	SubsumptionModule.java	38

4.3.2	Sensor Inputs	41

4.3.3	Main Program	41

4.3.4	Testing	42

4.4	Subsumption Architecture	43

4.4.1	Level Zero	43

4.4.2	Level One	44

4.4.3	Level Two	46

4.5	Summary	47

Evaluation	48

5.1	Level Zero	48

5.2	Level One	48

5.2.1	Obstacle Avoidance	49

5.2.2	Random Wandering	50

5.3	Level Two	51

5.4	Hardware Evaluation	53

5.5	Software Evaluation	53

5.6	Summary	54

Conclusions	55

6.1	Sensors	56

6.2	Robot	56

Appendix A	58

Appendix B	59

Appendix C	62

Appendix D	63

References	65

Table of Figures

Figure 2.1 SRF08 ultra sonic range finder(left) and Sharp GP2D12 infrared range finder (right)	8

Figure 2.2 Front view of the CMUcam	9

Figure 2.3 Lego's RCX, the programmable brick [9]	12

Figure 2.4 Conversion table for RCX sensors, Mike Gasperi [18]	14

Figure 2.5 Three touch-sensor expander, Mike Gasperi [18]	15

Figure 2.6 Three touch-sensor expander conversion table	15

Figure 2.7 Example of multiplexer to expand RCX inputs	16

Figure 2.8 Multiplexer using sensor as clock	16

Figure 2.9 Interface from Sharp GP2D12 to Lego RCX, Philippe Hurbain [17]	17

Figure 2.10 A horizontally layered control architecture	20

Figure 2.11 A vertically layered control architecture	20

Figure 2.12 Levels of competence as proposed by Brooks [1]	21

Figure 2.13 A subsumption module, as suggested by Brooks [1]	22

Figure 2.14 Ballfaenger by Tim Rinkens	24

Figure 3.1 One of the caterpillar tracks on a Lego robot	27

Figure 3.2 The Lego twin front bumper	28

Figure 3.3 Proposed side view of Lego robot	28

Figure 3.4 Overview of execution of software	30

Figure 3.5 Level 0 architecture	30

Figure 3.6 Level 0 architecture augmented with level 1 modules	31

Figure 3.7 Level 1 architecture augmented with level 2 modules	31

Figure 3.8 Array of range finders, with ball in front of sensor B	32

Figure 3.9 Truth table for ball location given sensor pair comparisons	32

Figure 3.10 Hardware overview block diagram	33

Figure 4.1 Comparator circuit to compare a pair of ranges	35

Figure 4.2 Logic circuit to locate ball	35

Figure 4.3 Buffering the two outputs	36

Figure 4.4 Interactions between methods in SubsumptionModule.java	40

Figure 4.5 Flow diagram for the AVOID module	45

Figure 5.1 Example encounter with a wall	49

Figure 5.2 How the robot handled a corner	50

Figure 5.3 Example trace of the robot's movement around the arena in 3 minutes	51

Figure 5.4 The Lego robot moving towards a ball	52

Figure 5.5 Times taken to locate a ball in an empty arena (m:ss)	52

Table of Listings

Listing 4.1 Example of complex timed execution	41

Listing 4.2 Anonymous instantiation of a subsumption module	42

Listing 4.3 Connecting subsumption modules	42

Listing 4.4 Implementation of the BUMP module	43

Introduction

This project deals with the implementation of subsumption architecture on a low cost robot, namely one constructed from Lego. There has already been a lot of work done in the field of robotics, but invariably such projects cost a large amount of money and have quite advanced computational capabilities. This project will investigate whether subsumption architecture can be implemented on and successfully control a very cheap system.

The task that the robot will be programmed to perform is the location and tracking of a tennis ball in a simple environment with only walls and the tennis ball.

Subsumption architecture is a layered control system, and both the implementation and the testing of the system can take advantage of this. The system will be expected to avoid objects while locating the tennis ball, and this will be implemented as a lower level of control.

The robot will be tested by giving it various tasks to perform and measuring both how well it does them and, if applicable, how long it takes to do them. The tasks will include locating a ball in the middle of an arena, locating a ball at the side of an arena, and tracking a moving ball.

This report will first investigate existing practices concerned with robots, including their physical construction and possible control mechanisms. This information will then be used to design a solution to the task, which will then be implemented. The robot will then be tested as mentioned above, and finally the success of the project will be analysed, and some possible areas of development will be looked into.

Literature Review

This project is concerned with robotics and artificial intelligence. Robotics is the study of robots; the Collins English Dictionary [] defines a robot to be:

“Any automated machine programmed to perform specific mechanical functions in the manner of a man.”

Artificial intelligence is the ability of a machine or other device to perform tasks that are normally thought to require intelligence. The definition of intelligence is a matter of much discussion, especially when applied to artificial intelligence; the Collins English Dictionary defines it as:

“The capacity for understanding; ability to perceive and comprehend meaning.”

With reference to this project, intelligence can be thought of as the ability of an entity to make rational decisions concerning its environment and to learn from experience.

Robotics applications range from spray painting cars in an assembly line or moving items around warehouses to complex tasks such as landing on the surface of planets and exploring the terrain. The robot is composed of a number of key elements, starting with input sensors, which are essential to allowing the robot to learn information about its environment. Equally important are the methods of interacting with and moving around in the world, namely robotic arms, wheels or other manipulation devices. Finally, there is the question of instructing the robot how to go about its tasks. The simplest form is direct programming, where the robot carries out actions using its actuators as dictated by a series of requests from a simple program. More complex robot control architectures are being explored to make robots more autonomous and intelligent, which is where robotics meets Artificial Intelligence.

All three of these areas – input, output and architecture – are investigated in this section of the project, to provide options that can be applied to the problem.

## Input Sensors

A robot’s task involves interacting with the world around it, whether it be navigating through a room or painting the correct parts of an automobile. This interaction is assisted by any of a range of sensors that can be incorporated into the robot. There are a number of functions that sensors can perform: point range finding, light detection, acoustic monitoring, distance measuring, compass reading, and even temperature detection. A comprehensive collection of sensors can be found at the Robotics FAQ []. This project involves recognising and avoiding objects, and so the next few pages will look at the range finder, the most common type of sensor, and cameras.

### Range finders

A range finder detects the distance to the object at which the sensor is pointed. Using a number of sensors, or a number of sweeps with the same sensor in different directions, a rudimentary map of the area surrounding the robot can be constructed. If the robot then moves to a new location, it can combine data retrieved at both locations to build a more detailed map of the area. Range finders can also be used to protect the robot from bumping into objects – a range finder pointing directly ahead could be used to immediately halt the robot or make the robot take appropriate evasive action should an object appear in front.

The two main methods used for calculating distances are infrared and ultrasound. An ultrasonic range finder sends out a pulse of ultrasound, and measures the time until it detects the returning wave, which has been reflected by the target object. As the speed of the wave is known, the distance can be calculated reasonably easily. An infrared (IR) range finder uses a different method. An infrared emitter and a detector are positioned a known distance apart, pointing in the same direction. The emitter emits a burst of IR light, and the detector can measure the incoming angle of the reflected light. Knowing the angle of the incoming light means that simple trigonometry can be applied to provide the distance to the reflecting object. Infrared range finders are largely superseding ultrasonic sensors, as they have higher accuracy over a greater range.

The most popular infrared range finder used in amateur robotics is the Sharp GP2D12 [], a small, cheap device that can measure distances between 10cm and 80cm with acceptable precision, and which is substantially immune to light fluctuations and the reflectivity of the target. A popular alternative to the GP2D12 is Robot Electronics’ SRF08 [], a high performance ultra sonic range finder. Costing slightly more than the GP2D12, at £30 instead of £15, the SRF08 offers a higher maximum range of three metres.

Figure 2.1 SRF08 ultra sonic range finder(left) and Sharp GP2D12 infrared range finder (right)

When the GP2D12 is provided with a 7-volt supply, the output pin produces an exponential return voltage between 4 volts (for close ranges) and 0.5 volts (for distant objects). For ranges closer than approximately 10cm, the output voltage starts to behave erratically, falling off very quickly to a near-zero voltage. Beyond 80cm, the output voltage is a more or less constant 0.3 volts, although noise in circuitry connected to the sensor can cause oscillations in this value.

Converting the output voltage into a distance is best done by means of software; doing it electrically is possible, but this increases the amount of noise in the circuit, and can be very hard to tune correctly. Instead, a lookup table of values can be used or some conversion function can be worked out to perform the translation.

### Cameras

One of the main aims of robotics is to enable robots to see their surroundings in more detail, and extrapolate information from the images. To do this, a camera and significant computing power is required; mathematical morphology is the technique used to process images for objects.

Cameras can cost from £50 to £1000, depending on the quality of the image required and the physical size of the camera. In robotics, monochrome cameras are predominantly used, as the images are much easier to process for objects; they cost considerably less; and they are almost as good as colour cameras: colours can often be deduced from the intensity of the light, and the scene the camera is viewing. In any case, robotic vision is primarily concerned with the recognition of patterns in a scene, and this can be done with a monochrome camera as easily as with a colour camera.

One of the main drawbacks of using cameras is that although the camera itself can be very small, it will invariably require a large external box, such as a PC, to do the actual image processing, and a good power supply. Although wireless cameras are available, at a price, it is unlikely that the cost could be justified for this project, and the image processing could be a project in itself.

One camera that does not require an external computer to do the processing is the CMUcam [], which is less than 2” square, has a 1/4” wide-angle lens, and is cheap at only $100. The camera has an onboard microprocessor that performs simple vision recognition processes at approximately 17 frames per second. The results can be communicated to a PIC controller via a standard serial bus. As stated on their website, [] “The CMUcam vision system makes it possible to ignore the complexity of camera interfacing and use vision just like any other sensor (i.e. sonar) often used in robotic systems”. The CMUcam can be instructed to track the position and size of a colourful or bright object, acquire and track the first object it sees, measure the RGB statistics of an image region, and even dump entire bitmap images over the serial port. Regrettably, the CMUcam was only made available early in 2002, by which time this project was too far under way to permit the use of this wonderful piece of equipment.

Figure 2.2 Front view of the CMUcam

## Robot Movement

Actuators are robots’ other physical attributes – devices that allow the robot to interact with the world around it, and change its position in the world. The most important of these is of course the drive method, and there are three ways of driving the robot: tracks, wheels and legs.

### Legs

Probably the most difficult but most impressive way of making a robot move is by legs. Either two or four (or more) legs can be built and arranged with appropriate gear and cam combinations to provide a walking motion. However, robots built with simple legs are very unstable, and find it very difficult to get over even slightly rough terrain.

### Wheels

The simplest method for driving a robot is with wheels. The two most popular options are to have two independently driven wheels and one free-moving support wheel, or to have four wheels, a pair (or both pairs) of which can have their orientation changed through use of a rack and pinion gear arrangement.

Using independently driven wheels relies somewhat on the ability of the wheels to slip. If the rear wheels of a robot are powered and provide the steering, then to turn in that direction one wheel is driven in one direction and the other in the opposite direction. However, this means that the front two wheels must slip sideways across the ground, which opens up the possibility that they may catch on objects on the ground. There is also a potential loss in precision, as the amount of slippage cannot be guaranteed.

A rack and pinion arrangement is more accurate, but much harder to implement. The rear pair of wheels is used to drive the robot, and the front pair is able to turn to some extent using a rack and pinion system between them, powered by another motor. The disadvantage of this is that an extra input sensor to the robot is required for the robot to know the position of the front wheels.

### Tracks

The final method for driving a robot is using caterpillar tracks. On each side of the robot, two small but wide wheels are positioned some distance apart along the length of the robot and connected by a rubber belt with treads to grip the ground.

A robot with two tracks can be extremely manoeuvrable; if the caterpillar track on either side is independently driven then the robot can be made to rotate on the spot, with no slipping. Stability is excellent – the tracks are normally very wide, and close to the ground. The belts that comprise the caterpillar tracks can have ribbed grips, and of course the increased surface area gives excellent purchase on almost any surface. One downside of tracks is they leave the robot rather close to the ground, giving it poor clearance over any obstacles.

## Robot Construction

Building a robot usually consists of a very extensive building phase. It is desirable for autonomous robots to be robust and reliable, and hence the physical construction of the robot is crucial to its success. For robots that are going into space, such as those used in the missions to Mars, lightweight materials such as carbon fibre and titanium are used both for strength and weight reduction. This project is not overly concerned with the physical construction of robots, but rather with their ability to appear cognitive, and therefore such an extended building phase would be a severe drawback.

A programmable device known as the OOPic [] can be used in the construction of mobile robots. The OOPic is an object oriented programmable device, and up to 128 of them can be networked using only three wires. Each one has the capacity to store a program of about 2000 instructions, which can be written in a high level language such as Java or C++ and compiled specifically for the OOPic. The OOPic is also very easy to interface with other devices such as the CMUcam mentioned earlier, which makes it an excellent choice. However, the use of OOPics still leaves the physical construction of the robot to be done using other materials and as mentioned, this can be very difficult.

Luckily, there is a very popular alternative – the well-known children’s toy Lego []. More specifically, the Lego Mindstorms kit, which includes a “programmable brick”, sells considerably more to the adult and robotics market than to children.

## Lego

Lego has a number of excellent features that make it ideal for use in the creation of a robot such as the one required. It has a huge selection of gears, wheels and structural elements, so that it is very easy to make complex robots. This means there is no need for a long construction period of the robot, and that the project can focus on the control of the robot rather than the creation of the hardware. Lego’s modularity also means it is very easy to add extra functionality to an existing robot, be it more wheels, extra sensors or simply some structural enhancements.

Lego is very cheap; the most expensive Lego Mindstorms kit costs around £100, and that will be well suited for this investigation. It is also very robust; the blocks themselves are extremely hard to break, and simple construction techniques can make Lego models tough enough for the purposes of this investigation, although not strong enough for a more advanced application.

### Lego Mindstorms

The Lego Mindstorms system introduces a “programmable brick” known as the RCX to the available construction items. This is a large device that has an on-board processor, a 32Kb RAM, three input ports, three output ports and an LCD screen to provide feedback. Software and firmware are downloaded to the RCX using a low speed serial infrared link with the infrared (IR) tower connected to a PC, and can be programmed using Lego’s own software or one of a number of alternatives. The infrared link can also be used while the robot is in motion to communicate with a nearby computer, which has to be in the line of sight. Kekoa Proudfoot [] has created an IR tower interface, which can be used to help the RCX communicate with software on a PC. The RCX also has 4 buttons: a power button, a “run” button, a “view” button and a “program” button. These buttons have pre-defined actions in the standard RCX firmware, however they can be assigned to different tasks in replacement firmware.

Figure 2.3 Lego's RCX, the programmable brick []

Sensors are plugged into one of the three input ports, which use a special Lego connection system, and up to 3 motors are driven by the three output ports. Lego provides many input devices and a standard output motor. It is also possible to interface custom sensors with the RCX, as discussed in Section 2.4.3.

The Lego Mindstorms website [] includes a number of user-created Lego robots incorporating the RCX, ranging from 4-legged and 2-legged walking robots to robots which can play games of checkers. This shows the diversity and power of the RCX.

Lego’s provided method of programming the RCX is known as RCX Code, which is very simple and inexpressive. It involves the connection of different blocks together in a graphical interface; excellent for anyone who has no programming experience. However, its lack of variables, function calls and its inadequate conditional statements makes this method of programming the RCX very limited.

### Alternative RCX Firmware

The RCX has a 32K RAM into which can be downloaded one of a number of types of alternative firmware, to allow programming in a more complete and expressive environment. There are three main types of replacement firmware; NQC, TinyVM and LeJOS.

NQC [], or Not Quite C, has very similar syntax to C, and is hence easy to pick up for any experienced C programmer. NQC actually uses the same firmware as the standard Lego software. However, it allows users to program textually instead of graphically, and introduces structures not available in the Lego software. One of the drawbacks is that this means that NQC is limited to 32 variables, and structures such as arrays are not possible.

TinyVM [] is a Java based replacement firmware for the RCX, with a footprint of around 10Kb. This means that there is 22Kb left over for programs, and typically 16Kb of this will be available for a small program to access as RAM. It includes pre-emptive threads, arrays, exceptions, recursion, access to RCX buttons, and many other features. Unfortunately, TinyVM is very poorly documented, which makes it rather difficult to use it to program the RCX. It also lacks support of floating-point arithmetic, and string constants cannot be used.

LeJOS [] is a more complete Java firmware for the RCX; indeed, it is an extension of TinyVM by Jose Solorzano, one of TinyVM’s original authors, and a new group of developers. LeJOS has all the features of TinyVM, and the addition of floating-point operations, string constants and many other APIs. The documentation for LeJOS is also much more complete, and has a full set of API docs, which is very useful. Regrettably, it has the larger footprint in the RCX of 16Kb, which leaves only 16Kb for user programs and memory to be accessed by the RCX.

One of the main features of TinyVM and LeJOS is that they are programmed in Java, which is an excellent object-oriented language. The LeJOS APIs include a standard set of mathematical functions, and some interfaces for Buttons and Sensors which allow the user to specify what happens when a button is pressed or the value on a sensor changes. LeJOS also allows multiple threads to be used: a feature which is essential in real-time systems these days.

### Lego Sensors

As mentioned, Lego provides a number of sensors for use with the RCX. There are light-sensors, which convert the brightness of the light into a value; there are touch sensors, which are either on or off and can be used in a bumper to detect obstacles; there is the Lego Vision Command, a visual interface; and there are many others. These sensors are all very good for simple robots. Unfortunately, Lego provides no range finder, and the Lego Vision Command camera has severe limitations.

The Lego Vision Command package consists of a colour USB camera that can be mounted on a robot, which is connected to the USB port of a PC. The Vision Command software allows the user to program it in a variety of ways. The image can be split up into a maximum of 8 areas each of which can have an action attached to it. If movement is seen in an area, or if a certain colour is detected in an area, then a command can be sent to the RCX, via the IR tower, which the robot can then act upon.

The whole process is rather slow though. The image processing on the PC can take up to a second, and the time to send a message to the RCX via the IR tower makes Lego Vision Command an impractical solution, though interesting – the environment could have changed significantly by the time the RCX has started to respond.

The voltage on each sensor input of the RCX is converted into a raw value, which varies between 1023 for 5.0 volts and 0 for 0.0 volts. Additionally, each sensor input can be defined as either a Light, Temperature or Touch sensor, and depending on the input type selected this raw value can be converted into different values for use in the program. Alternatively, the raw value itself can be used in the program. Mike Gasperi [] constructed the table shown in Figure 2.4, which shows the conversion of a range of voltages into raw and different sensor values.

| Volt | RAW | Light | Temp C | Touch |
| --- | --- | --- | --- | --- |
| 0.0 | 0 | - | - | 1 |
| 1.1 | 225 | - | 70.0 | 1 |
| 1.6 | 322 | 100 | 57.9 | 1 |
| 2.2 | 450 | 82 | 41.9 | 1 |
| 2.8 | 565 | 65 | 27.5 | 0 |
| 3.8 | 785 | 34 | 0.0 | 0 |
| 4.6 | 945 | 11 | -20.0 | 0 |
| 5.0 | 1023 | 0 | - | 0 |

Figure 2.4 Conversion table for RCX sensors, Mike Gasperi []

Using this information, it has been possible for a number of homemade sensors to be developed for the RCX including noise activated sensors and sophisticated line following devices. The standard method for inputting analogue readings to the RCX involves connecting the ground wire of the RCX sensor lead to the ground of the circuit, and connecting the other wire to the analogue output of the sensor circuitry.

The main drawback of the RCX is that it only has three inputs, which would allow it a very poor view of the world. To overcome this, multiple sensors can be connected to a single input, using a variety of techniques. Mike Gasperi [] has compiled a number of methods, some of which are shown below.

### Multiplexing

If two touch sensors are simply connected to the same input then the value of the sensor input is simply the OR of both sensors – if one sensor is pressed then the value of the input becomes 1, otherwise it is 0. If the wires on two or more touch sensors are aligned so that the switches operate in parallel then the logical AND function of the sensors can be achieved – the sensor input only becomes a 1 when all touch sensors are depressed.

It is also possible to connect three touch sensors to one input and detect exactly which one has been pressed, if each touch sensor is connected to the sensor input through a different value of resistor, as shown in Figure 2.5. This creates a simple digital-to-analogue converter, which can be decoded by the program using the table in Figure 2.6 to work out which sensors are depressed.

Figure 2.5 Three touch-sensor expander, Mike Gasperi []

| Input 3 2 1 | RAW Value Range |
| --- | --- |
| 0 0 0 | 1023 - 1001 |
| 0 0 1 | 1000 - 955 |
| 0 1 0 | 954 - 912 |
| 0 1 1 | 911 - 869 |
| 1 0 0 | 868 - 830 |
| 1 0 1 | 829 - 798 |
| 1 1 0 | 797 - 768 |
| 1 1 1 | 767 - 0 |

Figure 2.6 Three touch-sensor expander conversion table

The final method is to electronically switch between different input sensors. One way of doing this is to use one of the motor outputs as a clock pulse to drive a multiplexing chip, which selects a different output on the clock edge. A number of sensors could be connected to the outputs of the multiplexer, and the motor output would be used to select which sensor was desired. Figure 2.7 illustrates this principle – it is incomplete and only provided to explain what the idea involves.

Figure 2.7 Example of multiplexer to expand RCX inputs

When the multiplexing chip on the left sees a clock edge, provided by an RCX motor output, the selected output goes high, and activates the AND gate to which it is connected. The input from the appropriate sensor is then propagated to the RCX input. This would only work for digital values; to expand it to work with analogue values would require the use of relay switches in the place of the AND gates; activating a relay allows whatever voltage is on the input to be passed through.

This method however requires one of the 3 RCX motor outputs to work, which in many cases would be acceptable, unless a third motor is required to add to the possible methods of interaction with the environment. Alternatively, the sensor input can be used to drive the multiplexer. When a sensor input has its type changed from Light to Touch, it becomes a passive input and so drops the 8 volt supply. This drop can be detected and used to drive the clock of the multiplexing chip. Figure 2.8 shows the circuit devised by Mike Gasperi [] to put this theory into practice.

Figure 2.8 Multiplexer using sensor as clock

The diode network on the left provides the dual usage of the sensor input; when the sensor type is changed, the clock input to the multiplexing chip is triggered, and the selected output changes. As before, this output is used to select one input device for propagation to the RCX, again through the diode network.

### Interfacing with Range Finders

Regrettably, the SRF08 ultra sonic range finder needs to be driven by an I2C bus, available on many popular controllers such as the OOPic, however absent on the Lego RCX. This means that the most attractive range finder for use with a Lego robot is the GP2D12, which has been successfully connected to the RCX by Philippe Hurbain [], using the rather complex circuit shown in Figure 2.9. The circuit overcomes the problem of requiring a separate voltage supply to the GP2D12 by using a combination of capacitors and a transformer chip, shown at the top of the diagram.

Figure 2.9 Interface from Sharp GP2D12 to Lego RCX, Philippe Hurbain []

The simplest way to interface the GP2D12 to the RCX however is to use an external power supply, such as a 9v battery, and just connect the output pin of the sensor to one of the sensor inputs, through a potential divider to lower the voltage going into the RCX. Converting the readings from their linear input to a distance could be performed through a lookup table, which would be fast and easy.

## Architectures for Intelligence

One of the key areas of robotics, which ties in with artificial intelligence, is how to program the robot. As mentioned earlier, the simplest method is direct manipulation, where the robot simply follows a set of commands programmed in by a human operator, or even mimics gestures made by a human using a control column. Such a robot cannot be considered intelligent, and has a very limited usage.

To give a robot a level of intelligence, some form of programming that can first process inputs from sensors that are monitoring the environment and take appropriate actions must be implemented, and there are many ways of doing this. For a long time architectures have provided a blueprint from which robotics designers can construct robots, and which provide computational services such as communication to the various subsystems within the robot. Such architectures tend to be very domain specific and poorly suited to adaptation to new environments.

In 1995, Woolridge and Jennings [] described an agent as:

“... a computer that is situated in some environment and that is capable of autonomous action in this environment in order to meet its design objectives.”

This description was extended by Lieberman [] in 1997 as follows:

“An agent should display some ... of the characteristics that we associate with human intelligence: learning, inference, adaptability, independence, creativity, etc.”

Applying this definition to robots, a robot can most certainly be thought of as an agent, which must autonomously survive in its environment. Three types of agents exist, logic based agents, belief-desire-intent agents, and reactive agents.

### Logic Based Agents

Logic based agents rely on creating a formal logical representation of the environment, and use logical deduction to reason about the environment. The representation of the environment will need to encapsulate objects that are of relevance to the robot’s survival and mission; actions that can be taken given certain conditions of the world; and implications which can be used to reason about other things given known conditions.

An example is a predator-prey situation. The predator can be programmed as a logic based agent, and given knowledge such as “approaching prey will cause it to run in the opposite direction”, “if you’re hungry, you need food”, and “prey will grow tired after X amount of time running”, can be released into a simulated environment to fend for itself. Funge et al [] have created a Cognitive Modelling Language, (CML), which can be used to program such agents. Actions can be defined, assigned certain preconditions and deemed undesirable. The expected response to certain actions can be given and many other conditions can be incorporated into the agent.

Logic based agents are potentially very powerful. They use good logical semantics to reason about the world, and can develop an understanding of the world relatively easily. The rather significant drawbacks however are that logical reasoning can take a considerable time, and indeed the decided course of action may no longer be applicable when the reasoning has been completed, because the environment has changed so much while the action was being computed. Also, the representation and set of reasoning about a complex environment can become very difficult indeed, resulting in a slow execution time.

### Belief-Desire-Intent Agents

Belief-Desire-Intent (BDI) agents are inspired by theories of how humans think. Beliefs are what the agent knows about its environment; desires represent courses of action that are currently available to the agent; and intents are the set of actions which the agent has chosen as optimal and has committed itself to. The desires are constantly updated, making use of new beliefs that the agent acquires about the environment and information about how well it is doing at accomplishing its chosen tasks.

Research in this area has so far been limited, and there is no standardised way of implementing or representing BDI agents. One of the major problems in the area is how to decide whether an agent is overcommitted to an intent that is not working, or alternatively whether it is not committed enough.

### Reactive Agents

The theory behind reactive agents is to refrain from modelling the environment to any level. Instead, the agent maintains very close contact with its surroundings. Intelligence is the product of the interaction of a number of programmed reactions to inputs, also known as “behaviours”. An example behaviour would be the seeking of a power source if the robot’s battery was running low. Such behaviours are accomplished by directly mapping sensor input changes to actions the robot should take. Some form of interaction between the different behaviours is necessary to ensure the correct action is taken, specifically if more than one behaviour decides on a course of action.

Reactive agents are inherently very fast and efficient, as there is no complex reasoning between sensor input and actuator output. This means there is very little time lag between the environment changing and the agent responding. They are in general very robust. The main conditions are that the inputs to the system must be of very high quality, and the environment local to the robot must have sufficient detail for the robot to choose an appropriate action.

The method of cooperation between behaviours can be of two forms, referred to as “vertical layering” and “horizontal layering”. With horizontal layering, each behaviour, or layer, is connected directly to the inputs and outputs, as shown in Figure 2.10. Each behaviour computes what it thinks to be the best course of action given the current situation, and sends this to the outputs. Obviously some sort of mediator is required in this situation, to decide which behaviour has given the most important, correct or intelligent result. This mediator can be very difficult to implement, and all possible combinations of outputs must be considered when the system is constructed.

Figure 2.10 A horizontally layered control architecture

The vertical layering method instead maps at most one layer to each input and output. With a one-pass control method, as shown on the left of Figure 2.11, this layer processes the input and passes it on to the next layer, which in turn makes its decision and passes on the result to the next layer, until finally some action is taken. A two-pass control method, as shown on the right of Figure 2.11, passes the input results up the layers, and then the computed results from higher levels are passed back down to the lower levels. This allows lower layers to forbid other layers from submitting a different course of action and prevent taking dangerous action. Vertical layering has the benefit that there are many fewer interactions to consider, and no complex mediator must be constructed, though it has the severe drawback that an error in one level will be likely to have serious consequences.

Figure 2.11 A vertically layered control architecture

### Subsumption Architecture

In 1986, Rodney Brooks [] proposed a task-orientated system of control for a mobile robot that he called subsumption architecture. In essence, the system breaks the problem down into a number of tasks, or behaviours, each of which believes it is the sole inhabitant of the world, with complete control over the robot. Interaction between the layers is achieved by tapping into data paths of other levels in the architecture, and suppressing the flow of data. This removes the need for a central control module, while maintaining the idea of a vertical layered architecture. He says that:

“The control system can be viewed as a system of agents each busy with their own solipsist world.” []

Brooks defines a number of levels of competence for a mobile robot, starting at object avoidance and exploration and finishing at the formulation and execution of plans, and ultimately reasoning about the behaviour of objects. These levels are shown in Figure 2.12.

| 0) | Avoid contact with objects (whether the objects move or are stationary) |
| --- | --- |
| 1) | Wander aimlessly around without hitting things |
| 2) | “Explore” the world by seeing places in the distance that look reachable and heading for them |
| 3) | Build a map of the environment and plan routes from one place to another |
| 4) | Notice changes in the “static” environment |
| 5) | Reason about the world in terms of identifiable objects and perform tasks related to certain objects |
| 6) | Formulate and execute plans that involve changing the state of the world in some desirable way |
| 7) | Reason about the behaviour of objects in the world and modify plans accordingly |

Figure 2.12 Levels of competence as proposed by Brooks []

This system can be applied to the construction of a control system for a mobile robot by creating the control mechanism in a series of layers, each of which implements a level of competence. First, a mechanism that achieves level 0 competence is created and debugged. This is never altered, and another mechanism is created, which is allowed to examine data from the level 0 system and suppress the flow of data through level 0 with its own information. This second layer is designed so that together with the first layer it achieves level 1 competence. The first layer continues to run unaware that there is anything happening elsewhere, even though its data paths are sometimes interrupted by the second layer’s messages.

Each layer is constructed from a network of finite state machines, which have been augmented with timers. Each finite state machine has input, output, suppression and inhibition signals, which are interconnected with other finite state machines. To allow the data flow between layers, a hierarchically higher level can inhibit the outputs of finite state machines in a lower level, or suppress the inputs to finite state machines with an altered value. Each module can be in a number of known states, computed as a function of its inputs, timer and current state. Outputs can be emitted if the module enters a certain state.

Figure 2.13 below shows how a subsumption module would be drawn. Inputs can be suppressed by another module; the number in the circle represents the length of time that the input is suppressed for. When an input is suppressed, the value of the suppressing output is taken by the input. Outputs can be inhibited; again, the number in the circle represents for how long. Inhibition of an output does not present a value for the input to which the output is connected.

Figure 2.13 A subsumption module, as suggested by Brooks []

This model is parallel, and a suitable system could be implemented to allow all the modules to operate independently of each other, either in software on a multi-processor machine or at the gate level in hardware. On a single processor system the complexity of the modules is so low as to not be a serious problem, and a simple scheduling algorithm would be able to decide what modules needed attention and which were idle.

The system is purely reflexive; actuators are controlled from direct observation of the sensors. The system also has no internal representation of the world it is in, which saves significantly in storage space. For this reason it is essential that all input devices are of the highest quality, and multiple readings of a single element of the environment with different sensors will be extremely useful to minimise errors and ambiguity.

While the idea seems very simple, Brooks [] explains that:

“Complex (and useful) behaviour need not necessarily be a product of an extremely complex control system. Rather, complex behaviour may simply be the reflection of a complex environment. It may be an observer who ascribes complexity to an organism – not necessarily its designer.”

Essentially, subsumption architecture provides a simple method of creating a control system for a mobile robot that will enable it to operate in a complex environment. In his paper in Architectures for Intelligence [], Brooks postulates that, with suitable, but not substantial, hardware upgrades, subsumption architecture could scale upwards significantly, and exhibit progressively more impressive cognitive behaviour. Indeed there is a school of thought that believes that human behaviour is very similar to that described by Brooks, but with a much higher level of competence, and with considerably better input and output apparatus.

## Existing Projects and Work

The number of projects which have been created with Lego is enormous – everything from chess playing robots to dancing and musical robots. LeJOS is one of the main platforms of choice, and indeed a Lego robot named “Jitter” which ran LeJOS was recently sent to operate on the International Space Station.

### Subsumption Architecture in LeJOS

The LeJOS package actually comes complete with an example subsumption architecture written by Paul Andrews [], the object of which is to allow the robot wander around and avoid obstacles using left and right bumper listeners. The code is well written, and downloading it onto the RCX shows that it does in fact work.

However, the code is not very expandable: while it includes a Task class that can be extended to provide extra modules for a subsumption architecture, they do not follow Brooks’ specification very well, as modules cannot be linked together, and suppression and inhibition of connections is not possible. It does have the concept of finite state machines, but that is as far as the similarity goes.

### Tennis Ball Catcher

Tim Rinkens [] created a Tennis ball catcher from Lego and the RCX and implemented a simple program to locate and catch balls using LeJOS. The software is rather simple, although perhaps challenging to read as the description and all variables and method names are written in German, but the design of the Lego robot is ingenious. Figure 2.14 shows a computer representation of the “Ballfaenger”; of note are the grabbing jaws at the front, which are activated when the sensor lying between them (with two spikes down to the ground) detects a ball. The ball-bumper could be useful in this project, depending on whether or not it can be combined with other bumper sensors.

Figure 2.14 Ballfaenger by Tim Rinkens

### Brooks’ Robot

Also of considerable interest is the first robot to be equipped with a subsumption architecture, built by Brooks and his team [] in 1986. This robot was not constructed from Lego; instead it ran on three wheels and used a drive mechanism from Real World Interface. The robot was approximately 0.8 metres high and was equipped with a ring of 12 sonar range finders and 2 CCD cameras that could be used to view the environment. Processing was done by an off-board Lisp machine.

With level 0 competence programmed into the robot, it found an empty space and stayed in the middle. If it was approached by a person then it would back away. It was possible for two people to “herd” the robot in any direction, even down tight corridors.

When level 1 competence was programmed on top of this, the robot would generate a random heading every few seconds if it detected it was not moving anywhere. This meant that it was no longer content sitting in the middle of a room, and would instead wander aimlessly.

When level 2 competence was added, the sonar sensors were used to detect a corridor of open space in some direction, and the robot would head towards that space. If a person walked in front of the robot while it was trying to get to its target, the robot would change course to avoid the person, although it would still eventually reach the target. The architecture used by Brooks contained 14 subsumption modules in total, and contained many feedback loops, so that modules in the architecture could return information to modules earlier in the decision process.

### Map Building with a Lego Robot

Enrique Romero [] designed and constructed a Lego robot that was programmed with LeJOS to map an environment. He used a pair of GP2D12 range finders angled 45° from the front to follow walls and allow more accurate positioning and turning of the robot. He interfaced the sensors directly with the RCX sensor inputs, using an external 9v battery as the power supply.

The software used no specific architecture, but he was able to make the robot create a map of a simple indoor environment. His robot used wheels as the drive mechanism, with a pivot wheel at the front for support. A single front bumper was used for obstacle avoidance, and an odometer was used to measure the distance between objects.

The project was a success; the robot successfully mapped out complex environments and transmitted maps of the locations back to a PC over the IR link. This shows that the RCX can be used for advanced applications, although he too noted the problems with lack of RAM on the RCX, and the drawbacks of so few sensor inputs.

## Problem Definition

The aim of this project is to create a robot from Lego that utilises Brooks’ subsumption architecture [] to locate and track an object while showing some apparent level of cognition. While any object could be used, for the purposes of this project the object used will be a tennis ball: its size is approximately 1/4 the width of the front of a Lego robot, and the robot will be able to push it easily. A tennis ball is also not so large as to appear like a wall, which is necessary for the robot to distinguish it from the walls of the arena..

This will be a four-stage process, the first being the design of a Lego construction that will serve as the platform for the mobile robot. Next, it will be necessary to build sensors to enable the robot to locate a tennis ball and distinguish it from its surroundings. The next step will be to implement subsumption architecture on the RCX, and finally to create a subsumption architecture that can locate and track a tennis ball while taking care to service its lower level functions such as object avoidance.

Design

The design stage of the project can be broken down into a number of sections; these include the constraints placed on the project, and the software and hardware elements of the robot.

## Design Constraints and Limitations

A number of constraints exist both on the scope of the project and the capabilities of the tools. It is to be assumed that the robot will exist in a simplified environment. To exist in a complex environment with multiple objects of varying sizes it would be necessary to implement some sort of visual recognition, which is too complex a problem for this project. Most mobile robots also have a much larger selection of sensors that can be used such as gyroscopes, odometers and multiple range finders.

The robot is to be built with Lego – an adequate tool for the creation of simple mobile robots. However, the input capacity of the RCX means that the number of sensors is highly restricted. While multiplexing techniques have been discussed, they are deemed too complicated to implement in this project. To construct a working multiplexer would require an advanced hardware implementation stage, and this project is more interested with the implementation of subsumption architecture than with creating an impressive array of inputs. Lego parts also govern the speed of the robot, and the Lego motors are far from speedy (although they will be sufficient for this project).

The RAM size of the RCX must also be taken into consideration; it has 32Kb in total. Of this, up to 16Kb is normally occupied by the firmware leaving a mere 16Kb for the user’s programs and the storage space which that program can access. Most firmware also has quite significant restrictions. For example, NQC can only handle integers, and LeJOS cannot make use of long integers.

## Lego Robot Platform

The first stage of design concerns the Lego robot itself. Lego provides a number of solutions in the user manual [] that comes with the Mindstorms kit, so it seems prudent to start there.

Three base designs are provided; the acrobot, the roverbot and the inventorbot. The inventorbot is a two-legged robot that has various “human interaction” features such as hand shaking and head turning, and hence is rather inappropriate. The acrobot is an agile robot that has the ability to flip back and forth onto various springy attachments which cushion its fall.

The roverbot is the most practical of the 3 solutions, and there are designs for various drive mechanisms and bumpers provided which all attach neatly to the standard base. The three drive mechanisms available are legs, wheels and tracks. The legs are asynchronous and operate rather erratically: even if both motors are going at the same speed there is no guarantee that the legs will rotate with the same period. This causes the robot to sway from side to side and to veer from its intended course.

When the wheels were tried, they did not provide a long enough wheelbase. This meant that although the robot could turn very tightly it was very unstable and any extra weight near the front would tip it over. As the robot is required to have a ball sensor mounted on the front, this is highly undesirable, and hence the wheels are a poor choice. The tyres on the wheels were also made of rather poor rubber that skidded very easily, even on carpeted surfaces.

Caterpillar tracks are the final drive type. They are low and wide (about 2.5cm in either dimension). They stretch for the full length of the robot and are driven by a gear train. On full power the robot can cover a metre in approximately 10 seconds with very little deviation in course. If the motors are set to turn in opposite directions then the robot can turn on the spot doing a full turn in approximately 6 seconds. The tracks grip carpet extremely well – the robot requires a very firmly held hand blocking its path until it stops moving. Even then the tracks do not slip; it is the motors that stop operating.

One drawback of the caterpillar tracks is that the gears that power them are exposed and very close to the ground, making them very susceptible to the collection of fluff and dirt. HHHowever, in the controlled environment in which the robot will be operating, this is not much of a problem. The drawbacks of wheels and legged robots, and the benefits of tracks means that tracks are the drive mechanism of choice for this project.

Figure 3.15 One of the caterpillar tracks on a Lego robot

The input sensors are another issue that needs to be resolved. The three input ports to the RCX rather limit the choice of sensors, without the use of multiplexing, which has been deemed too complex for this project. Some method of detecting objects in front of the robot is necessary, and the two options are a forward facing range finder or a front mounted bumper. A range finder could be implemented using the GP2D12, though its accuracy at close range makes its usage a little problematic. Interference from other objects and other sensors could also cause problems for the robot. A bumper could be implemented instead, using the touch sensors provided by Lego with the RCX.

Designs for two different front bumpers are provided in the Lego Mindstorms user guide []; one spans the whole width of the robot, and the other is in two parts, with one bumper covering each side of the front of the robot as shown in Figure 3.16. The first is rather flimsy, and if an obstacle is presented to the far side of the bumper the robot can still continue for some way until there is sufficient pressure on the touch sensor in the middle of the bumper to register the collision. If the bumper pieces are turned around to face forwards, a sort of scoop is created, which could be helpful in ensuring the ball does not roll away.

Figure 3.16 The Lego twin front bumper

The second bumper uses two touch sensors to form two independent bumpers, one on either side of the front of the robot. The construction means that the bumpers are a lot more stable, and they are much quicker to respond to collisions. The fact that the RCX can tell on which side of the robot the collision has occurred also means that better avoidance tactics can be implemented.

Figure 3.17 Proposed side view of Lego robot

In addition, it must be possible to mount the sensor for detecting balls from a distance on the front of the robot. The design of the robot base means that it is very easy to mount the circuit boards, and there is plenty of room for the actual sensors. The most obvious place would be as shown in Figure 3.17: directly above the front bumper and angled slightly downwards, so that the maximum range of the sensor would intersect the floor. This would mean that a ball close to the robot would probably go unnoticed. Nevertheless, hopefully by the time the ball is directly ahead and the robot is heading for it the robot will be able to remember that the ball is there, and respond accordingly when the ball apparently disappears from sight.

## RCX Software Design

From the beginning, it was clear that the standard RCX firmware was insufficient for the realization of this project, and so a replacement firmware had to be chosen. The best candidate for this is LeJOS. TinyVM is now discontinued, and has a lot of missing features; NQC is incomplete and still limited to 32 variables. LeJOS, on the other hand, has excellent sensor handling and the elegance of Java object-orientated programming. The loading of LeJOS onto the RCX was very simple and was performed using a couple of commands and the IR tower. Once installation was complete, a couple of test programs were compiled and loaded onto the RCX to ensure everything was operating correctly, and to get a feel for the software.

The subsumption architecture could be implemented in software as a simple class. The class needs to have a method which defines how the module changes state and an array of inputs and outputs that can be connected together in any of the three forms proposed by Brooks: regular, suppression and inhibition (see Section 2.5.4).

The main method would then instantiate an array of these subsumption modules, and connect together the inputs and outputs appropriately. Sensor listeners would be created and connected to the subsumption architecture, and button listeners too, if need be.

The main method would then enter a tight loop in which it would try to execute each module in turn. If any of the inputs of the module had been updated since the module was last executed, the module’s execution method would be called and outputs updated accordingly. The flow of the main method can be seen in Figure 3.18.

If any timed execution needed to happen, such as complicated avoidance manoeuvres, then the execution method of the subsumption module could create a new thread which contained timing information and provision for the commands to occur at different intervals. If need be the thread could be interrupted by the subsumption module to which it was attached.

LeJOS can only support the concurrent running of up to 8 threads, and sensor and button listeners both start new threads running. This means that the number of timed execution modules would be limited, unless another method was devised.

Figure 3.18 Overview of execution of software

## Subsumption Architecture

The subsumption architecture being used for the project would best be designed from the ground up, as proposed by Brooks []. This meant that the robot needed to obtain level 0 competence, then level 1, and so on.

Level 0 competence as stated by Brooks is simply “Avoid contact with objects (whether the objects move or are stationary)”. Because the robot is using touch sensors in the form of bumpers to detect objects, some contact with the object is required before it can avoid it; the robot must just ensure that when it encounters an object it does not try to progress further in that direction, as it will be unable to move. This means that a sensor listener must be created for the front bumper which informs a “REVERSE” module when the bumper is hit. The REVERSE module then determines how to avoid the object, and tells the “MOVE” module what to do. This is illustrated in Figure 3.19; there are two connections between each module so that information about which side has been bumped can be transmitted. For layer 0 this would only involve backing away from the object. Once layer 0 is implemented, it needs to be tested fully, and then layer 1 can be implemented.

Figure 3.19 Level 0 architecture

Layer 1 needs to make the robot wander aimlessly around without hitting objects. The “WANDER” module is always running, and directs the robot to turn in some direction for a random time, and then go forwards for a random time, after which the module will start again, turning the robot.

The “AVOID” module hijacks the outputs from the BUMP module in layer 0 and combines this with the heading generated by WANDER to generate a new heading on which to send the robot. If no bumping has occurred, then the AVOID module just propagates the heading through, but if a bumper has been hit then it will reverse and try to get around the obstruction. The heading generated by AVOID is set to suppress the inputs to the MOVE module on layer 0, so that the new, more advanced avoidance tactics will override those accounted for in level 0.

Figure 3.20 Level 0 architecture augmented with level 1 modules

Level 2 competence can then be added on, allowing the robot to explore by “seeing” a tennis ball and heading for it. Once a ball has been detected, a “PATH” module works out the best way to get to the ball, namely in what direction it should set off, making use of the timed execution threads available in the subsumption module. This then inhibits the WANDER module from generating any new random headings for the robot, and suppresses the input to the AVOID module with the newly generated heading, which will ensure that any objects the robot hits will be avoided.

Figure 3.21 Level 1 architecture augmented with level 2 modules

This architecture, shown complete in Figure 3.21, should allow the Lego robot to move around in a slightly simplified environment, apparently hunting down tennis balls and nudging them around.

## Ball Sensor Design

The robot relies on the ability to detect a tennis ball from some distance. The most obvious and reliable method would be to use a video camera imaging system, which could take pictures of the environment and locate distinguishing features, such as a tennis ball. However as mentioned earlier, this is not possible for a project of this scale. If the CMUcam had become available a few months earlier, it would have been the perfect candidate for this project, but this was not to be.

So instead, it was decided to use multiple parallel range finders mounted on the front of the robot, and a combinatorial logic circuit to compare distances and reason about the presence of a tennis ball. Figure 3.22 shows the possible configurations for what an array of three range finders mounted on the front would see, with the ball in front of sensor B (labelled from the left), and different slopes of wall (or of course no wall) behind the ball.

Figure 3.22 Array of range finders, with ball in front of sensor B

If the readings from each sensor pair are fed into a simple comparator circuit, then the numbers between each sensor pair are generated; a “1” means that the sensor to the right is measuring a greater distance than the one on the left, and a “0” means the two distances are equal or the distance on the left is more.

If all possible permutations are generated, as shown in Appendix A, then the truth table shown in Figure 3.23 is produced. The two columns on the left show the possible values for the comparisons between sensors A and B and sensors B and C. The columns on the right show where, given the currently observed values on the sensors, the ball is located.

| A/B | B/C | B | A or C | Unsure |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |

Figure 3.23 Truth table for ball location given sensor pair comparisons

When the ball is in front of sensor B, the comparators give a very definite reading of 01. When the ball is in A or C, depending on the slope of the wall behind, different values may be recorded. A simple combinatorial logic circuit can take the values from the pair of comparators and produce three outputs which follow the truth table shown above.

To generate a value that can be read by the RCX, the outputs from the combinatorial logic must somehow be converted into an analogue value, by passing each output through some potential divider and then into the RCX. The only problem with this is that any output that is at 0 will act as a ground, and hence computing the correct resistor values will be impossible; however, the use of tri-state buffers will eliminate this problem. If each output is made to activate a tri-state buffer, then the outputs of the buffers can be connected to a resistor network to generate a different output voltage depending on which output is currently high. Figure 3.24 shows a block diagram representing the entire circuit.

Figure 3.24 Hardware overview block diagram

## Summary

In summary, the robot is comprised of four section; the ball sensor, the robot itself, the software and the subsumption architecture. A number of limitations need to be taken into consideration, including the limited size of the RAM on board the RCX, the lack of certain facilities in LeJOS, and the use of Lego as the construction tool.

# and chips

Implementation

It was possible to realise the robot’s various stages of development somewhat concurrently: implementing subsumption architecture did not rely on having a working robot, or indeed a working ball sensor. This section will discuss the issues concerned with the implementation of the four areas of the project: the ball sensor, the Lego robot, the implementation of subsumption architecture modules in LeJOS and the implementation of the decided subsumption architecture using these LeJOS classes.

## The Ball Sensor

As explained in Section 3.5, the ball sensor will consist of an array of parallel range finders, and some circuitry to compare the distances they are currently reading. The best sensor for the job, given its ease of use, is the Sharp GP2D12. Construction of the ball sensor went through two attempts; in the first, it was intended to try to detect a ball in the periphery of the view of the sensor, in front of either of the side sensors. Unfortunately, this became too difficult: the range finders did not provide a clean enough signal for the comparison circuits to make a definite decision on which sensor was closer, and hence a ball in the periphery would not produce a constant signal.

Instead it was decided to simply detect when a ball was present in front of the middle range finder. The circuit relies on using some simple comparators to decide which sensor in each pair is closer. The results from this are fed into some simple combinatorial logic, and the result is passed to the RCX through a buffer.

### Comparators

Comparators essentially take two voltage inputs and on the output pin return a voltage describing which one is larger, and, if required, how much larger. The two pins are referred to as the “inverting” and the “non-inverting” pin; if the inverting pin has a higher voltage then a negative voltage is put on the output, if the non-inverting pin is higher then a positive voltage is output. Figure 4.25 shows how one of the comparators was set up so that if the left hand sensor of a pair is measuring a shorter distance, then the output from the comparator is positive, and otherwise it is 0. A capacitor is placed between the output from each sensor and ground, to attempt to eliminate noise.

If two copies of the circuit shown in Figure 4.25 are created, one comparing the left-most sensor and the middle sensor, and the other comparing the middle and the right-most sensors, then a 2-bit binary output is produced, as discussed in Section 3.5.

Figure 4.25 Comparator circuit to compare a pair of ranges

### The Logic Stage

This 2-bit binary output can be fed into a small combinatorial logic circuit, which will make a different output go high for each different possible input combination. The two states that need to be tracked are LM and LM, where L refers to the comparison between sensors A and B, and M refers to the comparison between B and C. This can be realised using the circuit shown in Figure 4.26.

Figure 4.26 Logic circuit to locate ball

### Buffering

The final stage before passing the reading to the RCX is to buffer the output from the logic stage, and provide a different analogue value for each of the two possible outputs. If each output from the logic stage is used to activate a tri-state buffer, changing it from the high-impedance output (Z) to 1, then when the output is not active, it will not draw any current, which means it can effectively be ignored.

To distinguish between the two different values, one of the outputs from the buffer is passed through a potential divider, which presents between the two resistors a voltage which is half that used as the reference voltage, as shown in Figure 4.27.

Figure 4.27 Buffering the two outputs

Finally, the output from the circuit shown above is connected to a sensor input of the RCX, the GP2D12 sensors are fitted to the front of the robot, and the board containing the circuitry mounted on the rear of the robot. A picture of the completed ball sensor, before mounting on the robot, can be found in Appendix D.

### Testing

The RCX was then loaded with a simple program to display the raw value from one of the sensor inputs on the LCD display. Theoretically, when the ball is directly ahead of the robot, the display should show 1023, when the ball is to one side it should show around 500 to 600, and when no ball is present a 0 should be shown on the display.

Regrettably, the sensors were too noisy, or the comparators not accurate enough, and so when the ball was anywhere except directly ahead of the robot, an ever fluctuating value was shown. Plugging the output into an oscilloscope showed the same problem. Attempts to bias the sensors or comparators were unsuccessful, and no improvement could be made. It was therefore decided to settle on detecting the ball only when it was directly ahead, which provides a minor but manageable setback.

A number of test runs were then created for the ball sensor, to see how accurate it was. The array of sensors on the front of the robot was angled down at an angle that meant the maximum range of the sensor was approximately 40cm. If a ball was placed in front of the middle sensor within this range, then the RCX displayed that it had been able to detect a ball, as desired.

If the ball was placed within this range, and a wall was also within this range, then the sensor still detected the presence of a ball, no matter at what angle the wall was. The only time the ball sensor failed was when the ball was in a corner, with a wall on either side towards the robot. In this case, each sensor read the same distance, and hence it did not detect a ball in front of it. In any case, if the robot did detect the ball in a corner there is nothing it can do to dislodge it. Creating a robot capable of dislodging a ball in a corner would be a possible area for expansion, however it is not feasible with a Lego robot or the time available.

## The Robot

The quickest part of the project was the physical construction of the robot with Lego. The Lego Mindstorms kit comes with a user manual, which provides detailed instructions on how to build the “roverbot”, the chosen base for the project. Initially a solid centrepiece is constructed, which contains the motors, supports the RCX and provides places to connect tracks and bumpers to. Tracks are similarly easy to build and snap on to the base, and already a functioning robot is ready. A simple test program was downloaded onto the RCX to test basic actions.

Next the twin front bumper was built using the instructions in the user manual, and fitted to the front of the robot. At this point the textbook parts of the robot were complete, and all that remained to construct was a place to support the breadboard containing the circuitry for the ball sensor, and the range sensors that comprise the ball sensor itself.

Slight modifications to the rear of the robot provided a platform for a single breadboard, partially held down by the RCX itself and partially by other Lego pieces. The GP2D12 sensors could be secured to short Lego beams using twisted wire, and these beams were connected to longer ones to provide three sensors in the same plane, all pointing in the same direction. This construction was then mounted to the RCX by the use of an angled axle-connector, which was secured horizontally to the robot at one end and angled upwards towards the front of the robot, and finally connected to the ball sensor constructed earlier.

Finally, wires from the sensors were passed into a connecting block tied to the front of the robot, and then more wires were run under the RCX the length of the robot to be connected into the breadboard at the rear. Pictures can be found in Appendix D.

The robot was then loaded with a simple test program (programmed using Lego’s own programming environment for the RCX) that told the robot to go forwards all the time, unless one of the two bumpers was hit. If the left bumper was hit then the robot should reverse, turn right 90 degrees, and then go forwards again. The inverse was done for the right bumper.

Placing the robot in a complex environment such as a cluttered office space with chairs and tables tested the functionality of the tracks and bumpers very well. As intended, the robot reversed and turned away from any objects it hit. The bumpers were very responsive; collision with any part of the bumper would result in the robot reacting instantly. The only problems occurred when an object too low to impact the bumper and yet too high for the tracks to get over was run into, or there was an obstacle above the level of the robot. These two minor problems are, however, insignificant and acceptable.

Running on a normal carpeted surface, the robot could be guaranteed to turn through the same angle if the motors were put in opposite directions for a certain amount of time. When running in a straight, unobstructed line, little to no deviation from the intended course was observed. The tracks and gears did pick up a moderate amount of dirt and hair during a five-minute test period, however this did not noticeably affect the running of the robot.

## Software Implementation

Before creating the subsumption architecture discussed in Section 3.4, it was necessary to design a generic subsumption architecture in LeJOS, which could be used to create the desired architecture.

The implementation of the software went through many iterations. While the initial design and underlying ideas were sound, the RCX is limited to 32K of RAM, of which 16K is consumed by LeJOS. In fact, it transpires that not all remaining 16K is available for user programs; some is used up by the stack and general storage. Over the course of the project, it became apparent that the maximum program size that can be downloaded to the RCX is around 10K, when compiled.

This caused significant drawbacks: the first version of the software was very verbose and “user-friendly”, but only two or three subsumption modules could be instantiated before the entire RAM was consumed. This description of the software will make note of features of the code that were at one time implemented, and explain how they were removed, and what they were replaced with. It should be noted that a significant amount of time was spent optimizing chunks code so that they had the same function, just a different implementation that was less RAM intensive; such changes will not be commented on in this report, because the optimizations were simply clever code modifications and not of academic interest.

Initially, a small Timer class was made, which could be accessed by any other classes to get the current time of the robot, as counted in “ticks”. Each time all the modules were executed, the main class could call the Tick() method, which would increment the time. Any other method could call the Time() method, which would return the current time. The alternative to this was to use the RCX’s onboard system time However, testing showed that it appeared to be correctly implemented, and at any rate it ran to far greater precision than was required.

### SubsumptionModule.java

The main class of the software is the SubsumptionModule class. As discussed in the design section, the class would need the following features.

- An Execute method which would be called to update the state of the module

- Inputs and Outputs that could be connected to other inputs and outputs, and designated as suppressors or inhibitors as appropriate.

- The ability to allow an internal to be used by the module

When the class is instantiated, the constructor requires a number of inputs and outputs to be passed, and it initialises an array for each, of the correct size. The main method of the class is then the Run() method, which is called for each module in a tight loop. This method checks a flag to see if the module has had any inputs updated since it was last executed, and if so then it executes the Execute() method of the class, which is defined for each instantiation of the class. The complete listing for SubsumptionModule.java can be found in Appendix B.

Inputs

The inputs and outputs from a subsumption module are defined as member classes of the SubsumptionModule class. The input requires three internal variables and three methods other than the constructor. The variables keep track of the current value of the input, whether or not the module has been updated since the input was last updated, and if applicable a suppression time. The simplest method is Value(), which returns the current value of the input; to ensure the input was not corrupted by bad calls, the value variable was declared private.

The other two methods are Suppress() and Update(). Suppress()takes a time, in number of ticks, and a value to suppress the input with, and Update()simply takes a value. Suppress() first checks to see that the input’s last value has been processed, by checking the updated flag, and if so updates the value with the suppression value, updates the input and module flags to indicate they have been updated, and sets the suppression time to the current number of ticks plus the time passed as a parameter.

The Update() method also checks that the last value of the input has been processed, and then additionally checks that the time stored as the suppression time is less than the current time. If it is, then the value is updated. However, if it is not then the passed value is discarded.

Outputs

Outputs are very similar to inputs. They store an inhibition time, which indicates for how long they are inhibited, and a pointer to the input they are connected to, or an output they are inhibiting. There are three main methods to the Output class, two of which are overridden.

The ConnectTo() method is overridden with different parameters; one taking an Input object and one taking an Output object. If the Output object version is called, then this output is going to be used to inhibit the output that it is connected to, and if the Input version is used then the output will either update or suppress that input. The initial implementation ensured that an Output could only be connected to an Input or an Output, but not both, but this check was removed to save previous bytes.

The Update() method again has two different forms. One simply takes a value, and the other takes a value and a suppression time. If the one-parameter version is called, then a check is performed to ensure that this output is not inhibited at the current time, and then either the Output that it is inhibiting is inhibited for the time passed as the parameter, or the Input that it is updating is updated with the value passed as the parameter. If the two-parameter version is called then the first parameter is used as the new value for the Input to which the output is connected, and the second is used to suppress that input, through calling the Suppress() method of the input.

The final method is the Inhibit() method, which is called by an Output which is wanting to inhibit this output. The inhibition time is passed as a parameter, and is added to the current time to inform the Output when it can next be used.

It should be noted that passing 0 as the inhibition or suppression time to an input or output will cause any suppression or inhibition currently in place to be nullified. This can be useful for releasing the data paths once a module has finished with them.

The interactions between the subsumption module class and its two member classes, Input and Output, can be seen in Figure 4.28.

Figure 4.28 Interactions between methods in SubsumptionModule.java

Timed Execution

For timed execution to occur, a TimedExecution class is used, which is an extension of Thread. When the Run() method for a class which requires timed execution is first called, a TimedExecution object is created, which starts a new thread running to handle the execution of that module. In the event that an input of the module updates while the timed execution is part way through, the thread can be interrupted by the Execute() method of the class, and started from the beginning.

In practice, it turned out that the TimedExecution class was not acceptable, for two reasons. Primarily, the creation of so many extra threads was a severe resource drain, and only a very basic subsumption architecture could be implemented, with three or four modules, and one or two TimedExecutions. Secondly, LeJOS can also only support 8 threads at any one time, and it appeared that when the method for interrupting a TimedExecution was used the thread was not reused, but another one was created and used in its place, meaning that after 8 executions of the timed module the RCX would produce an error and crash.

Therefore a less resource intensive, however more conceptually complex, system was needed, essentially building timing checks into each of the modules that required timed execution. The new method is illustrated with pseudo code in Listing 4.1. In addition it is necessary to allow the Run() method of the Subsumption Module to execute the Execute() method of this timed module without any inputs being updated, and so an extra flag is added which allows this to happen.

if (input updated) {

executionTime = currentTime;

do first step;

}

else if (timerStart + firstTimeStep < currentTime) {

do second step;

}

else if (timerStart + secondTimeStep < currentTime) {

do third step;

}

Listing 4.1 Example of complex timed execution

### Sensor Inputs

Initially it was intended for sensors inputs to be implemented as Sensor Listeners, a LeJOS class that would extend the Java Thread class and could run concurrently. A class called Sense was created which on instantiation would connect itself to the input of a Subsumption Module object. The stateChanged() method, which is called by LeJOS when the value of the input is changed, would call a custom method that could decide if the new input value was significantly different from the old one. Once it had been decided if the new input was important for the subsumption architecture, the input to which it was attached would be updated.

In practice, this theory worked perfectly except when a large subsumption architecture was built and there was no room for extra threads, objects or classes. Therefore, because the Sensor Listeners took up too much room on the RCX, it was decided to simply allow subsumption modules to call the getValue() method of the appropriate sensor to get the current value. The software executes fast enough for each sensor to be checked regularly, and the space saving is significant.

### Main Program

The main program’s purpose is to create the subsumption modules with their individual Execute() methods, to connect the inputs and outputs together, and then to execute each module in succession, in a tight loop.

The instantiation of the subsumption modules makes use of a concept called anonymous inner classes. What this essentially allows is the overriding, and indeed general extension, of a class during its instantiation. Listing 4.2 shows an example of how a SubsumptionModule object could be created with a different Execute() method, some extra declarations, and an extra initialisation step.

modules = new SubsumptionModule[1];

modules[0] = new SubsumptionModule(2,4) {

extra declarations;

{ alwaysRunning = true; }

public void Execute() {

// this module's execution method;

}

};

Listing 4.2 Anonymous instantiation of a subsumption module

The first line creates an array of one subsumption module, the first and only element of which is then made into a new subsumption module with 2 inputs and 4 outputs. The bracketed statements following this declaration include extra declarations of variables, the initialisation of the alwaysRunning flag to true, and the overriding of the Execute() method to allow the subsumption module to perform its required task. Such anonymous instantiation of classes saves space, and makes for more understandable code.

The main program then needs to connect inputs and outputs of modules together, using the commands shown in Listing 4.3.

modules[0].output[0].ConnectTo(modules[1].input[0]);

modules[0].output[1].Inhibit(modules[1].output[0]);

Listing 4.3 Connecting subsumption modules

Now when the first output of the first subsumption module is updated, it will pass the value along to the first input of module 2. If the output were updated with two parameters, it would suppress the input instead. When the second output of module one is updated, it will inhibit the first output of the second module for a given time.

The final task of the main program is to initiate a tight loop in which the Run() method of every subsumption module is executed, after which the architecture’s time is incremented, and the execution stage started again.

### Testing

The testing of the implementation of subsumption architecture on the RCX took place in several stages. Initially the classes were created to work without the need for the LeJOS APIs, and instead printed output to a terminal window. This allowed the software to be debugged on a computer, without the lengthy download time to the RCX.

The full subsumption architecture, with sensor listeners and timed executions, was first implemented and run on a computer, with a very simple network of subsumption modules to test that all the methods worked correctly. It was checked that the inhibition and suppression worked correctly, and that timed execution threads did what they were supposed to do. Everything ran perfectly, and as expected.

As stated previously, the problems started to occur when the RCX was used to run the code. First, the inadequacy of the system clock came to light, and so it was necessary to implement a separate timer class. Then the problems with extraneous threads manifested themselves, and after a period of bafflement caused by LeJOS’ poor and undocumented communication of error messages, they were removed and replaced by the simpler method described earlier.

Finally, the RAM limit started to impede progress, and so various debugging and communication calls from within subsumption modules, which would display information on the RCX’s LCD screen, had to be removed.

Of course all these problems only became apparent when the full subsumption architecture designed in Section 3.4 was implemented. Some of the communication routines were left in place during the testing of the earlier stages of the subsumption architecture.

## Subsumption Architecture

Subsumption architecture lends itself very nicely to a progressive implementation. While the final subsumption architecture had seven modules, there are a total of three levels, with no more than three modules in each. The subsumption architecture was therefore implemented in three stages, and each level of competence was tested before the next stage was programmed.

### Level Zero

Level zero competence is intended to allow the robot to avoid objects, while giving it no ideas about how to move. As shown in Figure 3.19, this requires three modules in total, a BUMP module, a REVERSE module, and a MOVE module.

The BUMP module is very easy to implement. Two outputs are required which will be updated with the value read in from either of the two front bumpers. The two bumper sensors need to be initialised as well, telling LeJOS that they are touch sensors operating in Boolean mode. Listing 4.4 shows the portion of the main program that implements the BUMP module.

Sensor.S1.setTypeAndMode(1,0x20);

Sensor.S1.activate();

Sensor.S3.setTypeAndMode(1,0x20);

Sensor.S3.activate();

modules[0] = new SubsumptionModule(0,2) {

{ alwaysRunning = true;}

public void Execute() {

output[0].Update(Sensor.S1.readValue());

output[1].Update(Sensor.S3.readValue());

}

};

Listing 4.4 Implementation of the BUMP module

The REVERSE module requires two inputs, which are connected to the outputs of the BUMP module, and two outputs, which send movement commands to the MOVE module, one for each motor. The module checks its inputs for updates, and if either of them suggests that the robot has hit something with that bumper, the caterpillar track on that side of the robot will go into reverse for a set amount of time.

The MOVE module takes two inputs, one for each caterpillar track, and works out if the new value means that a change in speed or direction is required for one of the tracks. If a motor is already going forwards at full power, and another command to go forwards at full power is sent to it, it momentarily pauses, which is undesirable. The MOVE module therefore needs to maintain knowledge of which way each track is going, and only update it if necessary. To instruct the module to put one of the motors into reverse, a negative value should be passed to the input. A zero means that that motor should be stopped, and a positive value instructs the motor to go forwards. The magnitude of the value passed is the power that the motor should be set to, between 1 and 7. If a larger value than this is passed an error will occur. However, the error checking had to be removed to save space.

The final stage of the program, before the execution loop is entered, is to connect the modules together in the same manner as shown in Listing 4.3.

Once this level had been compiled and loaded onto the RCX, it was set in a large empty space and turned on. As expected, it remained stationary unless one of the bumpers was activated, in which case it reversed accordingly. If both bumpers were pressed then the robot would reverse, otherwise it would turn backwards away from the bumper that had been hit.

One obvious problem was that the robot could get backed into a corner, and it would not be able to sense that there was an obstacle behind it. A more complete robot could have multi-directional range finders, or even just a rear bumper, to solve this problem.

### Level One

Once level zero competence had been achieved and tested, the level one modules could be programmed on top of this. These modules are intended to generate a random heading for the robot, and combine a heading generated by a higher level of competence, such as the WANDER module, with the warnings from the bumpers to generate a new heading.

The WANDER module is defined as being always running, and it sends out a random turn command on its two outputs for a certain amount of time, followed by a forwards command. After a random amount of time going forwards, another turn command is issued.

The AVOID module needs to monitor the bumpers, and provide appropriate avoidance action if an obstacle is hit. Otherwise, it simply needs to propagate onto the MOVE module a heading passed to it, suppressing the current inputs to it. The way the architecture is implemented in LeJOS requires that an extra two outputs be added to the BUMP module, which can be plugged into this AVOID module.

The way the AVOID module works requires it to be always running, and to use internal counters to track the state of its execution. If the AVOID module senses a bump, then it should reverse, then turn away from the obstruction about 30 degrees, and then go forwards again. If it senses too many bumps in quick succession, then it should reverse away from the obstruction, turn, and then stop. At this point it will wait for a new heading to be sent from the WANDER module, which it is hoped will turn it away from whatever obstruction is causing so many collisions, such as a corner. The flow diagram in Figure 4.29 shows how the module described above works.

Figure 4.29 Flow diagram for the AVOID module

Testing of this level of the architecture is more complex than for level zero, and will be covered in more depth in the Evaluation. To ensure that the program was theoretically correct, it was implemented and tested using the subsumption architecture modelled in Java running on a computer, with simple communication with the standard output to show that the WANDER and AVOID modules were behaving as expected. Creating the optimal solution for the AVOID module is rather complex, and a complete listing can be found in Appendix C.

### Level Two

The final level of subsumption architecture that is implemented in this project is level 2, which should allow the robot to search out items or places that are desirable. This is done using the ball sensor, the construction of which is detailed in Section 4.1.

There are two modules needed to deal with detecting a tennis ball; the BALL module watches the ball sensor for changes, and informs the PATH module when a ball is detected.

The input that is to be used for the ball sensor is first set to raw mode, so that the voltage that it returns will be converted into a value between 0 and 1023. The BALL module checks this sensor input every tick, and if it is the within the same range as the value last tick, a count is incremented. If this count hits a certain level, experimentally defined to be 10 ticks, then the PATH module will be alerted to the presence of a ball. This means that any spurious signals will be accounted for, and ignored. If the PATH module has been informed of the presence of a ball, and the sensor loses track of the ball, then the BALL module will inform the PATH module that the ball has been lost.

When telling PATH that a ball has been found, the outputs from WANDER are also inhibited, so that no more random headings will be generated until the ball has been found or lost.

When the PATH module is informed of the presence of a ball, it needs to guide the robot towards it. This is done simply by suppressing the inputs to the AVOID module in level one, which means that level one can still handle any bumps which occur. The PATH module additionally starts a timer when it starts to chase a ball, and will halt the robot for a few seconds once it has been heading towards a ball for a given time, indicating that it has found it.

If the PATH module is told that it has just lost a ball, it must look for it somewhere. The first option is to reverse a little, in case the ball has come close enough to the front of the robot to become hidden from the ball sensor. The next alternative is that the ball has travelled to one side of the field of view of the ball sensor, and so the PATH module needs to turn the robot to either side to see if it can relocate the ball.

During initial testing, it was found that if the ball had indeed moved to one side of the robot, for example if it was rolling in front of the robot, then the robot could not turn fast enough to relocate it. For this reason, and the fact that there was no more RAM on the RCX, the “relocation” part of the PATH module was removed. If the PATH module sees a ball then it heads for it for a certain time, and then stops, and if it loses the ball then it surrenders control of the AVOID module’s inputs, and allows WANDER to take over movement again.

## Summary

The implementation of the robot was not all done at the same time; different sections could be completed without the need for the other sections at all. The construction of the robot itself was the easiest step as its design came predominantly from the Lego User Manual. The subsumption architecture was also easy to implement, once the generic subsumption module classes had been written for LeJOS.

The coding of the various modules on the RCX was very tedious: a lot of errors would occur for no apparent reason, and hours would need to be spent sieving through the whole of the code just to find out that the reason why it was failing was because it was running out of threads. A number of rewrites of the code were necessary, and indeed the initial working version of the SubsumptionModule class was twice the size of the final version.

More time could have been spent perfecting the ball sensor, but after weeks of tweaking the circuitry there was no noticeable improvement, and any more progress would have required a serious redesign of the ball sensor.

Evaluation

The entire robot having been constructed, it was now possible to evaluate the three levels of competence implemented. Each level was loaded onto the robot individually, and the results are documented in the following sections. The test arena was a 2.5m x 1.5m area of carpeted space, bordered by boxes and walls. In some of the tests, a 0.8m x 0.8m obstruction was added to the center of the arena.

## Level Zero

As stated by Brooks [], the initial level of competence for a mobile robot is “Avoid contact with objects (whether the objects move or are stationary)”. With this level of competence, the robot is programmed to reverse away from an object whenever a bumper is depressed. If the left bumper only is depressed then the robot should reverse to the right, and similarly for the right bumper. If both bumpers are depressed then the robot should reverse in a straight line.

This level is extremely easy to test. The robot was placed in the middle of an empty space, and the program executed. When the either bumper was activated the robot behaved as expected – indeed the robot could be persuaded to turn a complete circle if one bumper was held down for long enough.

Problems occurred when the robot was reversed into an object; with no backwards-looking sensors, the robot kept trying to reverse into the object. This was to be expected, however, and could be solved with improved sensor capabilities.

## Level One

Level one competence is characterised by random exploration of an area, and the ability to service level zero tasks, namely obstacle avoidance. This was implemented in this project by randomly generating alternating turn and forwards commands for random amounts of time, and using another module to avoid objects when they were hit.

There were two things to be checked for in this level, the obstacle avoidance techniques, and how well the random wandering was implemented.

### Obstacle Avoidance

When the robot encounters an object and one of the bumpers is depressed, it should reverse away from the obstruction, turn approximately 30° away from the object, and then go forwards again. If four bumps happen in quick succession, the robot should stop, as it is most likely trapped in a corner, and the wandering behaviour should restart within a few seconds to redirect it in a random direction.

Encountering Walls

The robot was placed pointing towards a wall and set to drive into it at a slight angle. The bumper depressed as expected, the robot reversed approximately 6 inches from the wall, and then turned to the side away from the bumper which had been hit for about 30°. It then proceeded forwards again, until it next encountered the wall, a couple of feet on. Again it reversed, turned and headed towards the wall again, at a much more gentle angle than before. The final impact was handled the same as the other two, and the resulting heading of the robot was almost parallel to the wall – it did not exactly follow the wall as might have been expected. This is explained by the fact that the turns were not exactly 30° each and that the robot did not hit the wall exactly head on to begin with.

An example encounter with a wall can be seen in Figure 5.30.

Figure 5.30 Example encounter with a wall

Stuck in a Corner

To test how well the robot could escape from a corner, it was placed pointing towards a wall with one bumper slightly closer to the wall than the other. The effect of this bumper being pressed would cause the robot to turn towards another wall, so that the opposite bumper would then be pressed. Under normal conditions, this could cause the robot to bounce between the sides of the corner indefinitely, but a corner escape mechanism has been implemented (see Section 4.4.2).

As expected, the robot reversed from the first wall and pointed itself towards the other wall of the corner, which it approached, collided with and reversed from, putting it almost back where it started. This was then repeated, and on reversing from the second wall, the robot stopped. A few seconds later a new turn command was generated by the WANDER module, and the robot turned away from the corner, effectively escaping it.

Occasionally the heading generated by the WANDER module was the forward command, which sent the robot back into the corner. After four bumps it stopped again for another command from the WANDER module, which would hopefully send the robot away from the corner. Figure 5.31 shows an example of how the robot escaped from one of the corners in the test area. The robot’s first point of contact is labelled 1, where it can be seen that the robot reverses and collides again three times in a row. It then reverses to point 5 where it stops until a new heading is generated by the WANDER module, sending it away from the corner.

Figure 5.31 How the robot handled a corner

### Random Wandering

The roaming techniques were measured by observation of the robot over a set period and by tracking the course it took. These observations showed the robot to follow the perimeter of the test arena to a greater extent than it did the middle of the arena. However, on average the robot was seen to cover the area sufficiently. The tendency to follow walls can be put down to the technique used to avoid walls, which invariably left the robot moving away from the wall at a small angle. Figure 5.32 shows an example session in the arena, lasting approximately 2 minutes. The robot is started near one wall pointing into the centre. The first command from the WANDER module is a turn command, and hence the robot moves towards one of the side walls, where it collides and successfully navigates away from the wall. As can be seen, each encounter with a wall tends to result in two or three manoeuvres to escape the wall; this could be basis for some improvement of the wall avoidance module.

Additionally, the robot was placed in the arena with an extra obstacle introduced: a large box in the middle, with a corridor down either side approximately 1.5 times the width of the robot. The robot managed to handle the environment surprisingly well, succeeding in navigating down the side of the box almost every time it tried, although there were two problem occasions.

On one occasion, the robot headed directly at one of the outer corners of the box, so that the corner was directly between the two bumpers. In this case neither bumper was depressed, and hence the robot kept trying to go forwards. Eventually the robot skidded around to one side, and one of the bumpers collided, causing the robot to perform the correct avoidance technique.

Figure 5.32 Example trace of the robot's movement around the arena in 3 minutes

The other failure occurred when the robot hit the side wall of one of the corridors, at such an angle that when it reversed and turned it was almost directly against the opposite wall, which caused it to reverse and turn again, effectively backing the robot out of the corridor.

## Level Two

The final level of competence achieved by this investigation is level two: the ability to “explore” an environment by locating objects and heading for them, in this case a tennis ball.

The robot has been equipped with a “ball sensor” which uses three parallel range finders on the front of the robot, and some hardware which reasons about the differences in ranges over the sensors. The robot is programmed to head directly forwards when the hardware detects that the middle range finder is pointing at a ball, and stop after a few seconds time, to indicate that it has found the ball. The robot must also service bumps from the lower level, and must maintain its random wandering capabilities when it is not in pursuit of a ball.

The situations of interest, and which will be evaluated, are: a ball in the centre of the arena, a ball against a wall and a moving ball. The same test arena as described earlier was used for this stage of the testing, without an object in the centre of the arena.

Ball in the Open

The robot was placed near one of the side walls, facing into the centre of the arena, where a regular tennis ball was placed. The location of the ball was such that it was out of range of the ball sensor on the front of the robot.

The robot was activated as before, and randomly wandered around the arena. After approximately 2 minutes of wandering, its path took it across the centre of the arena, where it detected the ball. As expected, the PATH module kicked in, informing us of the presence of a ball on the LCD panel. The robot then proceeded towards the ball, until it made contact. Rather unexpectedly, the robot then stopped, without signalling that it had found the ball. On closer inspection of the implementation of the subsumption architecture the reason was obvious: the BALL module had lost sight of the ball because it was under the vision of the ball sensor, and hence the PATH module had made the robot stop and wait for a new heading from the WANDER module.

Figure 5.33 The Lego robot moving towards a ball

On some occasions, when the robot detected the ball from further away, it approached the ball and stopped just before it, displaying that it had located the ball on its display, as expected. Figure 5.33 shows the robot approaching a successfully located ball.

Ten tests were run with the robot and the ball starting in the same location; the results can be found in Figure 5.34. On average, the robot located the ball in 3 minutes and 4 seconds. Given that the robot can travel a metre in approximately 10 seconds, and making allowances for turning times, it can be therefore deduced that the robot covered approximately 15 metres on average before locating the ball. The circumference of the entire arena was eight metres, and hence this means the robot completed the equivalent of two and a half laps of the arena before finding the ball.

| 2:34 | 2:12 | 1:54 | 4:15 | 3:12 |
| --- | --- | --- | --- | --- |
| 3:56 | 2:21 | 3:15 | 4:02 | 2:56 |

Figure 5.34 Times taken to locate a ball in an empty arena (m:ss)

Ball by a Wall

The next test was to see if the robot could detect a ball that was next to a wall. Again the robot was positioned to one side of the arena, pointing inwards, and the ball was placed against one of the long side walls, to one side of the robot.

The results of this experiment were slightly better than when the ball was in the open – as the robot had a tendency to follow walls, as was found in Section 5.2.2, it tended to find the ball considerably faster than when the ball was in the centre of the arena.

Occasionally, when the robot was travelling parallel to a wall, the ball sensor would not detect the ball, and instead it would drive right past it. When the robot approached the wall at an angle however, it was much more likely to see the ball, particularly when it collided with the wall, reversed, and then turned towards the ball.

Moving Ball

When the robot was presented with a moving ball, it did not perform so well. The robot was first activated and then placed, while moving forwards, in the middle of the arena. A ball was then rolled past the front of the robot, from one side to the other. Ideally, the ball sensor should have recognised the object and informed the PATH module. However, clearly its response time was too slow, and so the robot failed to perform any course correction. At any rate, the robot had been deemed too slow to turn and follow a moving ball, and hence any action taken by the PATH module would have been insufficient.

## Hardware Evaluation

The hardware of the robot was comprised of two things: the Lego robot platform and the ball sensor. The Lego part of the hardware performed admirably; the bumpers on the front were extremely sensitive, the tracks moved the robot along at a reasonable speed and with great reliability, and the general construction of the robot was very robust and secure. The robot was very expandable too, as was hoped from the beginning. While the initial robot was constructed directly from the User Manual provided with the Lego Mindstorms kit, it was a very simple task to add a platform at the back to support the circuitry, and to mount the ball sensor on the front of the robot at an appropriate angle.

The ball sensor was not quite as successful. While the premise behind the ball sensor was sound, and in practice it worked as expected, it did not provide enough information for the subsumption architecture. The robot was unable to detect balls that were a few inches off its centre line, even though it was capable of detecting balls directly ahead of it; the robot’s field of view was insufficient.

## Software Evaluation

The main aspect of this investigation is the usage of subsumption architecture on a Lego robot. The hardware that was running the architecture, the Lego Mindstorms RCX, was almost up to the task, although there were a few drawbacks.

The very limited RAM was a severe problem; there was no way a larger subsumption architecture could be implemented on the Lego RCX, unless the LeJOS firmware was altered to support a less complete implementation of Java, which would leave more space for user programs. The RCX’s execution of the subsumption architecture was however more than adequate. When the full program was loaded, it was estimated that the RCX was executing each subsumption module approximately 40 times per second – a statistic achieved by programming the robot to go forwards for 500 ticks and then stop, and then timing in the real world for how long it moved forwards.

One other potential way to overcome the RAM limitation of the RCX is to use multiple RCXs to execute different levels of the subsumption architecture, and enable communication between them through an IR link. This would allow a larger subsumption architecture to be implemented, hopefully allowing the robot a higher level of apparent competence.

## Summary

The evaluation of the robot turned up a number of points of interest. The software ran perfectly and as expected, after all the rewrites which were necessary to make it fit on the RAM of the RCX, and could quite easily be scaled up to implement a much larger subsumption architecture, given more RAM.

The robot as a whole was partly successful: it succeeded in locating a ball in an empty arena every time, and within an acceptable amount of time. It also managed to back away from obstacles, as required, and as such can be awarded a competence level of 2, as described by Brooks. Two hardware issues meant that the robot could not track a moving ball; the robot turned too slowly and the ball sensor could not detect objects in the periphery of its view.

Conclusions

This project set out to implement Brooks’ subsumption architecture on a low cost robot such as the RCX, and use it to locate and track an object such as a tennis ball. The project consisted of 4 main stages: the robot, the ball sensor, the implementation of subsumption architecture in LeJOS, and the creation of an appropriate subsumption architecture.

The project met with limited success; subsumption architecture was implemented very easily in LeJOS and, despite the RAM limitations, a successful architecture was created and loaded onto the robot. The ball sensor itself was rather poorly suited for the task required: it was created from moderate quality range finders and some clever logic, however it was not accurate enough to provide reliable information to the robot quickly enough. The robot itself was sufficient, however it was very slow moving and failed to track a moving ball.

The robot succeeded in locating a ball in an empty arena, backing off from any walls that it encountered, and escaping from corners in which it was stuck. As mentioned it failed to track a moving ball, but this can be put down to the robot’s very low speed relative to the speed of the ball.

It can be concluded that the main drawback of the project was the lack of reliable and detailed sensor information available to the robot, which shows that a reactive agent such as one implemented with subsumption architecture relies heavily on its inputs. Had better sensors been implemented, such as an onboard camera or range finders for detecting walls, then the robot would no doubt have achieved a higher level of competence with little difficulty.

The expansion of the Lego RCX into a more functional robot is unlikely for a number of reasons. The robot runs very slowly, it is not robust enough for more advanced applications, and the construction of larger robots with Lego would be very difficult. The RCX itself has very limited input and output capabilities, very poor amounts of memory, and processing power which is far more powerful than these drawbacks can make use of.

The Lego Mindstorms kit is perfectly capable of some tasks, but this project was pushing its capabilities, and anything more detailed would have been impossible to implement. A more expandable hardware architecture with better I/O devices should be used for the implementation of subsumption architecture.

It is clear that subsumption architecture is very good, and can be implemented with reasonable ease on a simple platform, but regrettably the Lego RCX is slightly too simple, with the limitations mentioned above. The implementation of subsumption architecture on a more expandable hardware architecture would be much more interesting to study.

There are a number of areas related to this project that could be improved upon, either as an extension or as an alternative.

## Sensors

The GP2D12 is an acceptable range finder, but its use as a detector for objects in a room is limited. The ball sensor constructed in Section 4.1 could be replaced with the CMUcam mentioned in Section 2.1.2. The only drawback of this is it is unlikely that the RCX could directly interface with the camera. This would mean that some intermediate controller, such as the OOPic, would be required to translate information from the CMUcam into sensor information for the RCX, and vice versa.

To create a proper subsumption architecture it is necessary to have a wealth of sensor information available for the modules, and so it would be a good idea to try other ideas of interfacing many sensors with the RCX, possibly by using a bus, as discussed below.

## Robot

While the RCX is potentially a very powerful platform for building mobile robots, it is severely limited by lack of RAM and poor I/O capabilities. It might be possible to remove the RAM chip from inside the RCX and replace it with a larger one, but such a task would not be easy.

Another option, as mentioned briefly earlier, is to use multiple RCXs to distribute the processing. A number of RCXs could be individually programmed, possibly to implement a level of the subsumption architecture each, and then networked together using some form of communication over the sensor ports.

This network communication could be tied in with a potential method to increase the number of inputs, using a bus to join them all together. One of the sensor inputs on each robot could be used as the bus line for sensors and RCX networking. Another sensor input could be used as a request line from both sensor modules and RCX modules. When a sensor or an RCX wanted to put some information on the bus it would need to apply for this to a master RCX, which would then respond (quite possibly using its motor outputs) and inform the sensor or RCX that it was allowed to use the bus. In addition the master RCX would use the final sensor input line to inform all the other RCX units what was currently on the bus, in case they needed that information.

Potentially that could allow for many more sensor types, and many RCXs all chained together. This could also be nicely tied in with the idea of subsumption architecture – each level could be implemented on a single RCX, which would use the bus to tap into the data paths of other levels. A controller module would be required, but purely to service bus requests.

An alternative would be to use a different system from the RCX. The main robot architecture could be implemented using the OOPic, as mentioned in Section 2.3, and interfaced with the RCX to provide control of the physical robot – namely the motors. Up to 128 OOPic devices can be networked together, each with up to 2000 instructions. Once again, this would lend itself very favourably to subsumption architecture. Each level could be programmed onto a separate OOPic, and could tap into the data lines of other modules over the bus.

Possible configurations of views from ball sensor (see Section 3.5)

SubsumptionModule.java (see Section 4.3.1)

//

//	SubsumptionModule.java

//

//	Created by Max Dyckhoff on Tue Nov 27 2001.

//

import MookExtras;

import java.lang.System;

import josx.platform.rcx.*;

/**

* SubsumptionModule

*

* Class representing a subsumption module. Initialised with a

* number of inputs and outputs, which are defined later as

* member classes. Every module is Run() in quick succession, but

* only if the module has had an input updated since the last run

* will the Execute() method be called, which contains the definition

* of what the module does, involving the checking of the inputs to

* the module and updating of the outputs.

*

* The Update() and Suppress() methods are provided to update or

* suppress inputs to the module - they are defined in this class so

* that they can be synchronized with the Run() method, ensuring that

* inputs are not changed while the module is doing its thing.

**/

public class SubsumptionModule {

protected boolean	moduleUpdatedFlag;

public 	 Input	input[];

public	 Output	output[];

private	 int		i;

protected boolean	alwaysRunning;

public SubsumptionModule(int numInputs, int numOutputs) {

input		= new Input[numInputs];

output	= new Output[numOutputs];

moduleUpdatedFlag	= false;

for (i=0;i<numInputs;i++) input[i] = new Input();

for (i=0;i<numOutputs;i++) output[i] = new Output();

}

public final synchronized void Run() {

if (moduleUpdatedFlag || alwaysRunning) {

this.Execute();

moduleUpdatedFlag = false;

for (i=0;i<input.length;i++)

input[i].inputUpdatedFlag = false;

}

}

public void Execute() {

;

}

/**

* SubsumptionModule.Input

*

* Member class representing an input to a subsumption module.

* Value is updated using the private methods Update() or

* Suppress(), which can only be called by SubsumptionModule, to

* ensure synchronization. If the input has already been updated,

* and the module has not been Execute()ed since, then the new

* input value is discarded.

*

* Update(VALUE)

*		- change the current value of the input to VALUE

* Suppress(VALUE, TIME)

*		- suppress the input for TIME with value VALUE

* Value()

*		- return the current value of the input

* ClearFlag()

*		- clear the updated flag, so the input can be updated again

**/

public class Input {

public	boolean	inputUpdatedFlag;

private	int		value;

private	int		suppressed;

public Input() {

inputUpdatedFlag = false;

}

protected void Update(int newValue) {

if (!(inputUpdatedFlag)) {

// the module has been executed since this input was

// last updated, so we can change the value of the input

if (suppressed <= MookExtras.Time()) {

value = newValue;

inputUpdatedFlag = true;

moduleUpdatedFlag = true;

}

}

}

protected void Suppress(int suppressValue, int suppressTime) {

if (!(inputUpdatedFlag)) {

// the module has been executed since this input was

// last updated, so we can change the value of the input

value = suppressValue;

inputUpdatedFlag = true;

moduleUpdatedFlag = true;

suppressed = MookExtras.Time() + suppressTime;

}

}

public int Value() {

return value;

}

}

/**

* SubsumptionModule.Output

*

* Member class representing an output from a subsumption module.

* Output has to be connected to another output or input, using

* one of two methods:

*

* ConnectTo(INPUT)

*		- connect this output to INPUT (for updating or suppression)

* ConnectTo(OUTPUT)

*		- inhibit the output from OUTPUT

*

* If this output is to be inhibited, then the Inhibit() method

* is called by the output doing the inhibiting :

*

* Inhibit(TIME)

*		- inhibit *this* output for TIME milliseconds

*

* The Update() method is used by the SubsumptionModule's

* Execute() method to change the value of an output. There are

* two methods, one for suppression and one for regular and

* inhibition.

**/

public class Output {

private int	inhibited;

private Input	connectedToInput;

private Output	connectedToInhibition;

public void Output() {

inhibited = 0;

connectedToInput = null;

connectedToInhibition = null;

}

public void ConnectTo(Input connectionPoint) {

connectedToInput = connectionPoint;

}

public void ConnectTo(Output connectionPoint) {

connectedToInhibition = connectionPoint;

}

protected void Update(int value) {

if (inhibited <= MookExtras.Time()) {

if (connectedToInput != null)

connectedToInput.Update(value);

else if (connectedToInhibition != null)

connectedToInhibition.Inhibit(value);

}

}

protected void Update(int value, int suppressionTime) {

if (connectedToInput != null)

connectedToInput.Suppress(value, suppressionTime);

}

public void Inhibit(int inhibitionTime) {

inhibited = MookExtras.Time() + inhibitionTime;

}

}

}

The AVOID module (see Section 4.4.2)

modules[4] = new SubsumptionModule(4,2) {

int bumped = 0;

int avoiding = 0;

int bumps = 0;

int bumpTime = 0;

{ alwaysRunning = true; }

public void Execute() {

if ((input[2].Value() == 1) || (input[3].Value() == 1)) {

if (bumped == 0) {

if (bumpTime + 100 > currentTime)	bumps++;

else											bumps = 1;

bumpTime = (int)currentTime;

}

if (input[2].Value() == 1)	bumped = -7;

else if (input[3].Value() == 1)	bumped = 7;

output[0].Update(-7, 25);

output[1].Update(-7, 25);

avoiding = (int)currentTime + 25;

}

else if ((avoiding < currentTime) && (avoiding > 0)){

if (bumped != 0) {

output[0].Update(-bumped, 20);

output[1].Update(bumped, 20);

avoiding = (int)currentTime + 20;

bumped = 0;

}

else {

if (bumps >= 4) {

output[0].Update(0, 50);

output[1].Update(0, 50);

bumps = 0;

}

else {

output[0].Update(7, 50);

output[1].Update(7, 50);

}

avoiding = 0;

}

}

else if ((avoiding == 0) && (input[0].inputUpdatedFlag ||

input[1].inputUpdatedFlag))

{

output[0].Update(input[0].Value(), 50);

output[1].Update(input[1].Value(), 50);

}

}

};

Pictures of completed hardware (see Sections 4.1 and 4.2)

The ball sensor before mounting on the robot, and the circuitry used with it

# References

[]	R.A. Brooks, A Robust Layered Control System For A Mobile Robot, IEEE Journal of Robotics and Automation, Vol. RA-2 No. 1., pp 14-23, March 1986.

[]	M. Woolridge and N. R. Jennings, Intelligent agents: theory and practice, Knowledge Engineering Review, Vol.10, No.2, pp 115-152, 1995

[]	Henry Lieberman, Autonomous Interface Agents, Proceedings of the ACM Conference on Computers and Human Interface, CHI-97, 1997.

Available from http://web.media.mit.edu/~lieber/

[]	Collins English Dictionary Millenium Edition, Haper Collins (Glasgow), 1998

[]	Lego Mindstorms Constructopedia, The Lego Group, 1999

[]	E. Romero, Localisation and Map Building for a Lego Robot, Department of Computer Science, University of York, March 2001

[]	Cognitive Modelling Language homepage, John Funge,

http://www.dgp.toronto.edu/~funge/software.html

[]	Lego Mindstorms website, The LEGO Group,

http://mindstorms.lego.com/

[]	Unofficial questions and answers about MIT Programmable Bricks and LEGO Mindstorms, Fred Martin,

http://fredm.www.media.mit.edu/people/fredm/mindstorms/

[]	NQC website, Dave Baum,

http://www.enteract.com/~dbaum/nqc

[]	TinyVM website, J.H Solorzano,

http://tinyvm.sourceforge.net/

[]	LeJOS website, Paul Andrews et al,

http://lejos.sourceforge.net/

[]	Robotics FAQ website, Kevin Dowling,

http://www.frc.ri.cmu.edu/robotics-faq/

[]	Sharp GP2D12 infrared sensor, Acroname Inc,

http://www.acroname.com/robotics/parts/R48-IR12.html

[]	SRF08 High Performance ultrasonic range finder, Robot Electronics,

http://www.robot-electronics.co.uk/htm/srf08.shtml

[]	CMUCam homepage, Carnegie Mellon University's Robotics Institute,

http://www-2.cs.cmu.edu/~cmucam/

[]	Interfacing the GP2D12 to the RCX, Philippe Hurbain,

http://philohome.free.fr/sensors/gp2d12.htm

[]	Mindstorms RCX sensor input page, Mike Gasperi,

http://www.plazaearth.com/usr/gasperi/lego.htm

[]	R. A. Brooks, How to build complete creatures rather than isolated cognitive simulators, in Kurt Vanlehn, Architectures for intelligence: the Twenty-Second Carnegie Mellon Symposium on Cognition, pp 225-239, Hillsdale, 1991

[]	IR tower interface, Kekoa Proudfoot,

http://graphics.stanford.edu/~kekoa/rcx/tools.html

[]	Subsumption architecture in LeJOS, Paul Andrews,

http://www.jools.net/

[]	Tennis ball catcher, Tim Rinkens,

http://rcxtools.sourceforge.net/model/e_model01.html

[]	OOPic homepage, Savage Innovations,

http://www.oopic.com/

A tri-state buffer has three states, 1, 0 and Z. When in the Z state, the output floats to whatever value is put on it by another device. The buffer has an extra input that states if the output should be in the Z state, or simply pass through the value on the input.

Normally to extend a class one uses the “SubClass extends Class” syntax, meaning that the new class has a name (“SubClass”). With anonymous instantiation, the subclass has no name, and is syntactically prettier.