Using the CDBOOT.EXE

1.)  Download the CDBOOT.EXE file to a blank floppy

2.)  On the working computer, Restart into DOS

3.)  Insert the Disk with the CDBOOT.EXE into the A: drive

4.)  Type      a:    			and press enter  (this will change to the a: prompt)

5.)  Type      CDBOOT.EXE    	and press enter  (this will self extract the files onto the a: drive)

6.)  Type      c:			and press Enter  (this will change to the c: prompt)

7.)  Type      sys a:		and press Enter  (this will transfer system files to the floppy drive)

Your bootdisk is ready for use !

To use the bootdisk

1)  Insert the bootdisk into the floppy drive of the computer you want to use the QuickCD for

2)  Connect the QuickCD to the parallel port of the computer and turn ON the power on the CD

3)  Restart the Computer and it should boot to the a: drive. After booting to the a: drive,

you should have access to the QuickCD.  If you have the c: drive only, then the QuickCD

would be assigned to the  D:  drive

4)  IF you do not have access to the QuickCD, then

*Make sure that your parallel port mode of the computer is set to EPP or Bi-Directional

mode.  This is done in the BIOS or CMOS of your computer.  You can contact the

manufacture of your computer to find out how to change your parallel port mode.