# Jeep® Cherokee Specs

## JEEP® Cherokee Technical Specifications

Powertrain

| Engine | 2.5-litre OHV I-4 | 4.0 litre OHV I-6 |
| --- | --- | --- |
| Horsepower, net brake @ rpm | 125 @ 5400 | 190 @ 4600 |
| Torque, net lb-ft @ rpm | 150 @ 3250 | 225 @ 3000 |
| Displacement, cc (cu in) | 2466 (150) | 3956 (242) |
| Bore x Stroke, mm (in) | 98.4 (3.88) x 81.0 (3.19) | 98.4 (3.88) x 86.7 (3.41) |
| Compression Ratio | 9.2:1 | 8.8:1 |
| Fuel System | Sequential multi-point electronic fuel injection |  |
| Transmissions | 5-speed manual with overdrive and synchronized forward gears; 10.5" hydraulically assisted, adjustment-free clutch (9.l" w/2.5L, 10.5" w/4.0L) 4-speed automatic with overdrive and lockup torque converter; electronically controlled |  |
| Gear Ratios, :1 | 5-speed manual (2.5L/4.0L) | 4-speed automatic (4.0L) |
| 1st | 3.93/3.83 | 2.80 |
| 2nd | 2.33 | 1.53 |
| 3rd | 1.45/1.43 | 1.00 |
| 4th | 1.00 | 0.75 |
| 5th | 0.85/0.79 | ------ |
| Reverse | 4.74/4.22 | 2.39 |
| Axle Ratio, :1 | 4.10/3.07 | 3.55 |

Four-Wheel Drive System

| Command-Trac® | Part-time, 2-speed with shift-on-the-fly capability; includes low-range (2.72:1); single lever transfer case mode operation |
| --- | --- |
| Selec-Trac® | Full-time, 2-speed with shift-on-the-fly capability; includes low-range (2.72:1); allows for 2- or 4-wheel drive mode, full or part-time 4-wheel-drive; single lever transfer case mode operation. Requires 4.0L engine and 4-speed automatic transmission. Not available on SE. |

Electrical System

| Alternator | 117 amps |
| --- | --- |
| Battery | 500 cold-crank amps |

Body/Suspension/Chassis

| Body Design | UniFrame construction--Body sheet metal and frame structural members welded and bonded into a single unit |
| --- | --- |
| Front Suspension | Quadra-Link® Live solid axle with 4 locating arms, coil springs, track and stabilizer bars, steering damper and gas-charged shock absorbers; beam axle on two-wheel drive models; 2500 lb Gross Axle Weight Rating |
| Rear Suspension | Live solid axle with multi-leaf springs, stabilizer bar, and gas-charged shock absorbers; 2606- to 2700-lb Gross Axle Weight Rating |
| Trac-Lok Rear Differential | Rear axle differential that locks in response to wheel slippage; includes heat-resistant synthetic axle lubricant |
| Steering | Power-assisted recirculated ball, 14:1 ratio |
| Turns | 3.4, lock-to-lock |
| Turning Diameter | 35.8 ft, curb-to-curb |
| Brake System | Front-Hydraulic, power-assisted low-drag ventilated disc brakes; composite steel; 11.0-in rotor diameter |
|  | Rear-Hydraulic, power-assisted self-adjusting drum brakes (9.0" x 2.5"); composite steel |
|  | Anti-lock brake system (Teves Mark IV-G)- Three-channel four-wheel full-time with four wheel speed sensors (requires 4.0L engine) |

SE Model Dimensions (inches unless otherwise specified)

Interior Dimensions

| Headroom, Front/Rear | 37.8/38.0 |
| --- | --- |
| Legroom, Front/Rear | 41.4/35.5 |
| Shoulder Room, Front/Rear | 55.3/55.8 |
| Hiproom, Front/Rear | 55.3/44.5 |
| Cargo Volume |  |
| Rear Seat Up/Rear Seat Folded | 34.4/71.8 |
| Seating Capacity | 5 |

Exterior Dimensions

| Length |  |
| --- | --- |
| Wheelbase | 101.4 |
| Overall Length | 166.9 |
| Overhang (to bumper) |  |
| Front/Rear | 27.5/36.5 |
| Width |  |
| Tread, Front/Rear | 58.0/58.0 |
| Overall Width | 70.5 |
| Height | 64.0 |
| Ground Clearance |  |
| Angle of Approach | 37.6° |
| Angle of Departure | 32.1° |
| Breakover Angle | 21.9° |
| Minimum Running Ground Clearance | 7.3 |
| Equipped with Up-Country Suspension Group | 8.6 |

Capacities/Weights

| Fuel Tank, gal (L) | 20.2 (76) |
| --- | --- |
| Cooling System, qt (L) |  |
| 2.5L engine | 10 (9.5) |
| 4.0L engine | 12 (11.4) |
| Base Curb Weight, lb |  |
| 2-Door, 2WD/4WD | 2891/3057 |
| 4-Door, 2WD/4WD | 2932/3100 |
| Payload Capacity, lb (with all occupants and cargo) | 1150 |
| Maximum Gross Vehicle Weight Rating, lb |  |
| 2-Door, 2WD/4WD | 4550/4850 |
| 4-Door, 2WD/4WD | 4600/4900 |
| Towing Data |  |
| Class III Towing |  |
| Gross Combined Weight Rating (2WD/4WD) | 8781/9060 lb |
| Maximum Loaded Trailer Weight | 5000 lb |
| Tongue Load Limit | 750 lb |

Trailer Tow Group Includes:

| Hitch | Load-equalizing hitch receptacle |
| --- | --- |
| Wiring Harness | 7 to 4 pin connector |
| Rear Axle | 3.55:1 with synthetic lube |
| Radiator | Heavy-duty |
| Fan | Auxiliary electric |
| Oil Cooler | Auxiliary transmission |
| Required Equipment |  |
| Engine | 4.0-litre I-6 |
| Transmission | Automatic |
| Spare Tire | Full-size |

TOWING RESTRICTION: Trailer frontal area limitation with maximum trailer weight of 64 square feet with 5000-lb trailer; maximum trailer length 25 ft; trailer sway control recommended.

This chart is meant to serve as a quick and easy reference guide on how to properly equip your Jeep Cherokee for towing. For specific details, however, discuss your plans with your Jeep dealer, who will help you select the right equipment to meet your specifications.

CAUTION: Never overload your vehicle beyond these design capacities. Chrysler Corporation cannot be responsible for brake performance if the Jeep vehicle and trailer hydraulic brake systems are in anyway connected. A separate brake system for all trailers weighing over l000 pounds gross is recommended. (State laws should be checked for trailer brake requirements.) See the Owner's Manual for important information regarding proper operation of this vehicle. See your dealer for additional equipment needs.

JEEP® Cherokee Equipment

1--Cherokee SE, 2WD or 4WD, Two- or Four-Door
2--Cherokee Sport, 2WD or 4WD, Two- or Four-Door
3--Cherokee Classic, 2WD or 4WD, Four-Door
4--Cherokee Country, 2WD or 4WD, Four-Door

Functional

|  | SE | Sport | Classic | Country |
| --- | --- | --- | --- | --- |
| Brakes Power-assisted, front disc/rear drum | S | S | S | S |
| Four-wheel anti-lock brake system (requires 4.0L engine) | O | O | O | O |
| Engine 2.5 litre (150 CID) 125 hp I-4 | S | - | - | - |
| 4.0 litre (242 CID) 190 hp High-Output I-6 | O | S | S | S |
| Engine Block Heater Electric | O | O | O | O |
| Heavy-Duty Electrical Package Includes 117-amp alternator and 500-cold-crank-amp battery | S | S | S | S |
| Quadra-Link Suspension | S | S | S | S |
| Side-Guard Door Beams | S | S | S | S |
| Skid Plates Transfer case, fuel tank, front suspension (included with Up-Country Suspension Group) (4WD only) | O | O | O | O |
| Speed Control Steering wheel-mounted | O | O | O | O |
| Steering Power assist | S | S | S | S |
| Tires P215/75R15AP all-weather black sidewall (4) | S | - | - | - |
| P225/75R15 Goodyear Wrangler® all-terrain outlined white letter (4) (requires conventional spare tire on SE) (Included with Up Country Suspension Group) | O | S | S | P |
| P225/70R15 Goodyear Eagle® GA outlined white letter (4) | - | - | - | S |
| Compact limited-duty spare, inside-mounted | S | S | S | S |
| Full-size spare, includes styled steel wheel | O | O | O | O |
| Trac-Lok Rear Differential (requires conventional full-size spare tire) | O | O | O | O |
| Transfer Case (4WD Models) Command-Trac part-time 4WD | S | S | S | S |
| Selec-Trac full-time 4WD system (requires auto. trans.) | - | O | O | O |
| Transmission 5-speed manual with overdrive | S | S | S | - |
| 4-speed automatic with overdrive and lockup features, (requires 4.0L) | O | O | O | S |
| Up Country Suspension Group Includes high-pressure gas-charged shock absorbers, P225/75R15 Goodyear Wrangler all-terrain outlined white letter steel radial tires, full-size spare tire; trac-Lok rear differential; Skid Plate Group; tow hooks (2 front and 1 rear) [if 4.0L engine i s specified, add heavy-duty radiator, auxiliary cooling electric fan (and auxiliary transmission oil cooler if auto. trans. is specified)]; also includes heavy-duty springs, unique front stabilizer bar (rear stabilizer bar is deleted) and rear window deca l (requires 4WD, and dual breakaway mirrors) | O | O | O | O |
| Wheels 15"x7" silver 6-spoke Styled steel with black hub covers (4) | S | - | - | - |
| 15"x7" silver Full-Face 5-spoke steel with argent hub covers (4) | - | S | - | - |
| 15"x7" 5-spoke Grizzly cast aluminum (4) | - | - | S | - |
| 15"x7" Lattice cast aluminum with bright hub center and Light Pearlstone or Gray Mist painted accents (4) | - | - | - | S |
| 15"x7" 10-hole sport aluminum with black hub covers (5) (requires conventional full-size spare) | O | O | - | O |
| Wiper/Washer Windshield, electric with two-speed wiper and variable intermittent features | S | S | S | S |
| Rear window wiper/washer with a fixed intermittent feature | O | O | S | S |

Exterior

| Bike Carrier | M | M | M | M |
| --- | --- | --- | --- | --- |
| Bodyside Striping Dual upper bodyside and liftgate striping | - | - | - | S |
| Bumpers Black with black end caps | S | S | S | - |
| Light Pearlstone or Gray Mist bumpers and end caps with black front bumper guards | - | - | - | S |
| Bright Package-Includes bright bumpers, bright dual power mirrors, bright door handles, grille, and headlamp bezels (requires Power Windows And Door Locks) | - | - | - | O |
| Fender Flares/Air Dam Black | S | S | - | - |
| Body Color | - | - | S | - |
| Light Pearlstone or Gray Mist | - | - | - | S |
| Fog Lamps Halogen | - | O | O | O |
| Glass Tinted, all windows | S | S | S | S |
| Deep-tinted door and rear quarter windows (rear quarter windows and vents on two-door) | - | O | O | O |
| Grille/Headlamp Bezels Black | S | S | - | - |
| Body Color | - | - | S | S |
| Lamps Halogen headlamps | S | S | S | S |
| Rear center high-mounted stop lamp | S | S | S | S |
| Mirrors Left/right, remote-control, black | S | S | S | - |
| Left/right, remote-control, black, with breakaway feature | O | O | O | S |
| Left/right, power remote-control, black (bright with Bright Package), with breakaway feature (requires Power Windows, Locks and floor console) | - | O | O | O |
| Moldings Black door window frames and dividers, windshield and liftgate window surround, drip rails, body-color roof | S | S | S | S |
| Black lower bodyside protection molding | S | - | - | - |
| Black lower bodyside treatment and protection molding with red accent | - | S | - | - |
| Body color lower bodyside cladding | - | - | S | - |
| Light Pearlstone or Gray Mist lower bodyside cladding | - | - | - | S |
| Roof Rack Adjustable, with tie-down provision, black | O | O | S | S |
| Ski Carrier | M | M | M | M |
| Spare Tire Carrier Outside | M | M | M | M |

Interior

| Air Bag Driver's. Always wear your seat belts for a fully effective air bag. | S | S | S | S |
| --- | --- | --- | --- | --- |
| Air Conditioning*** | O | O | S | O |
| Cargo Area Tie-down hooks (4) | S | S | S | S |
| Retractable cover, vinyl | O | O | O | O |
| Console Mini-Includes transmission shift lever (and transfer case lever on 4WD models) | S | S | - | - |
| Full-Includes armrest, storage box, rear ashtray, rear seat area heating ducts, double cup holder (and transfer case lever on 4WD models) | O | O | S | S |
| Overhead-Features: Display for compass, ambient temperature (in Fahrenheit or Celsius), map reading lights, and storage (Requires Power Windows and Door Locks) | - | O | O | O |
| Door Locks Rear, child-protection (four-door models) | S | S | S | S |
| Door Trim Color-keyed vinyl door trim with straight armrests | S | - | - | - |
| Color-keyed custom vinyl door trim with lower carpeting and hockey stick-style armrests | - | S | S | - |
| Color-keyed custom vinyl door trim with woodgrain upper garnish molding, lower carpeting and storage bin, hockey stick-style armrests | - | - | - | S |
| Floor Covering Color-keyed carpeting in passenger area and cargo area | S | S | S | S |
| Floor mats, front and rear, color-keyed | O | O | S | S |
| Heater/Defroster Climate control system with power ventilation and side window defrosters | S | S | S | S |
| Rear window defroster (required in New York state) | O | O | O | O |
| Instrument Cluster Gauge Group-Tachometer, voltmeter, oil pressure and engine coolant temperature gauges, low-fuel indicator light, trip odometer | - | S | S | S |
| Indicator lights-Direction/4-way hazard, high beam, 4WD mode | S | S | S | S |
| Speedometer, electric, and fuel gauge | S | S | S | S |
| Warning lights-Alternator, oil pressure, engine temp., seat belt, parking brake | S | S | S | S |
| Instrument Panel Cigar lighter/auxiliary power access | S | S | S | S |
| Cluster overlay, black | S | S | S | S |
| Cluster overlay, wood grain | - | - | - | S |
| Locking glove box with light | S | S | S | S |
| Utility bin below glove box | S | S | S | S |
| Light Group Includes dome/map light, cargo area light with tailgate switch, courtesy lights, engine compartment light, Sentinel headlamp delay-off switch, lighted vanity mirrors | O | O | O | S |
| Mirror Day/night rearview | S | S | S | S |
| Visor-mounted lighted vanity mirrors | O | O | O | S |
| Passenger-Assist Handles Roof rail-mounted, front passenger, and rear outboard | S | S | S | S |
| Power Windows and Door Locks Includes Infrared Remote-Keyless Entry System for doors and liftgate | - | O | O | O |
| Seat Belts Color-keyed lap and shoulder belt restraint system, front and rear outboard positions (lap belt only for rear center) | S | S | S | S |
| Seats Front reclining high-back bucket seats | S | S | S | - |
| Front reclining wingback bucket seats with adjustable headrests | - | - | - | S |
| Leather Seat Package-Includes reclining wingback leather bucket seats with vinyl trim and front seatback map pockets (includes power driver's seat) | - | - | - | O |
| Rear bench with folding seatback and removable seat cushion | S | S | S | S |
| Security System Locks and unlocks doors, turns on interior lights, and activates alarm; features remote transmitter (requires Power Windows and Door Lock Group) | M | M | M | M |
| Sound System AM/FM stereo radio with clock and two speakers (SE) | S | - | - | - |
| AM/FM stereo radio with four speakers | O | S | S | S |
| AM/FM stereo radio with cassette tape player, clock, and four speakers | O | O | O | O |
| Six premium Accusound by Jensen® speakers | - | O | O | O |
| Electronically tuned AM/FM stereo radio with compact disc player | M | M | M | M |
| Two rear ceiling-mounted speakers, includes cargo area light | O | S | S | S |
| Six-disc compact disc changer, cargo area-mounted | M | M | M | M |
| Steering Wheel/Column Black, soft-feel sport, 2-spoke | S | S | - | - |
| Black, perforated leather-wrapped sport, 2-spoke | O | O | S | S |
| Energy-absorbing column with anti-theft ignition | S | S | S | S |
| Tilt steering wheel | O | O | S | S |

S-Standard    O-Optional*    M-Mopar Accessory**    -Not Available    P=Package

*Some optional equipment is available only as part of an option group, or requires the purchase of additional optional model features. See your dealer for details.

**These and many other Mopar Dealer-Installed Accessories are available at extra cost to help you equip your vehicle to your individual needs. Many of these accessories can be installed prior to delivery of your new vehicle. Ask your dealer for a complete listing of genuine Mopar Dealer-Installed Accessories.

***Special engine cooling requirements for non-factory air conditioning. Failure to follow requirements may invalidate warranty. See your dealer for details.